Using a system disk to Flash the Firmware/BIOS for the SuperTrak66 Controller

Create a system disk from an existing Windows 95 or Windows 98 Operating System.

-From the Windows Desktop double click on "My Computer", right click on the "3 1/2 Floppy A:", Select "Format" from the menu.

-Select "full" from the top portion of the "Format 3 1/2 Floppy A:" dialog box and then select "copy system files" from the bottom portion of the "Format 3 1/2 Floppy A:" dialog box.

-Click on the "Start" button. When the formatting process is complete click on the "Close" button.

-Using WinZip or PkZip extract all of the contents from the SuperTrak66 BIOS/Firmware ZIP File to the System Boot Disk in the A: Drive.

-Reboot the machine using the System Disk. After reaching the A:> prompt type, “st66” (without quotations) and then press the “Enter” key.

This will launch a batch file that will automate the flash process for you. Do not kill the computers power or interrupt the flash process at all. Doing so will result in a RAID Controller that is UNUSABLE!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

Press any key when prompted and then restart the computer when the word “EXIT” appear on the screen.

Using a Windows 95b/98/98SE/ME Emergency/CDROM Bootdisk to Flash BIOS/Firmware of the SuperTrak66/100 Controller

-Boot the computer using the boot disk. At the Boot Selection Menu press the “F5” key to reach the A:> prompt.

-Type, “st66” (without quotations) and then press the “Enter” key.

This will launch a batch file that will automate the flash process for you. Do not kill the computers power or interrupt the flash process at all. Doing so will result in a RAID Controller that is UNUSABLE!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

Press any key when prompted and then restart the computer when the word “EXIT” appear on the screen.