### 1. EEPROM Layout

b15							b8	b7							b0

| ETHER_ID1 | ETHER_ID0 | 00H |
| --- | --- | --- |
| ETHER_ID3 | ETHER_ID2 | 01H |
| ETHER_ID5 | ETHER_ID4 | 02H |
| Reserved | MII_PHY_AD | 03H |
| SUB_SID1 | SUB_SID0 | 04H |
| SUB_VID1 | SUB_VID0 | 05H |
| Reserved | Reserved | 06H |
| Reserved | Reserved | 07H |
| Data_SEL | PMCC | 08H |
| AuxCurr | PMU_DATA_REG | 09H |
| Reserved | Reserved | 0AH |
| Max_LAT | Min_GNT | 0BH |
| BCR1 | BCR0 | 0CH |
| CFG_B | CFG_A | 0DH |
| CFG_D | CFG_C | 0EH |
| CHKSUM | 73H | 0FH |

### 2. EEPROM Content

| Word Offset (hex) | Bits | Default Value by EEPROM Utility (hex) | Name | Description |
| --- | --- | --- | --- | --- |
| 00~02 | [47:0] | 000000000000 | ETHER_ID | Ethernet address. Default is an invalid value. Customers SHOULD replace it with a valid Ethernet address. |
| 03 | [7:0] | 00 | MII_PHY_AD | PHY address. Default is an invalid value. Customers SHOULD replace it with a correct PHY address according to PCB layout. |
|  | [15:8] | 00 | Reserved | Reserved. Recommend using the default value. |
| 04 | [15:0] | 0000 | SUB_SID | Subsystem ID. Default is an invalid value. Customers SHOULD replace it with their own Subsystem ID. Subsystem IDs are vendor specific. |
| 05 | [15:0] | 0000 | SUB_VID | Subsystem Vendor ID. Default is an invalid value. Customers SHOULD replace it with their own Subsystem Vendor ID. Subsystem Vendor IDs can be obtained from the PCI SIG. |
| 06 | [15:0] | 3065 | Reserved | Reserved. Recommend using the default value. |
| 07 | [15:0] | 1106 | Reserved | Reserved. Recommend using the default value. |
| 08 | [7:0] | 1F | PMCC | Power Management Capabilities Control Register. |
|  | [15:8] | 10 | Data_SEL | PM data select register. |
| 09 | [7:0] | 00 | PMU_DATA_REG | PM data register. |
|  | [15:8] | 00 | AuxCurr | 3.3Vaux auxiliary current. |
| 0A | [15:0] | 0000 | Reserved | Reserved. Recommend using the default value. |
| 0B | [7:0] | 03 | Min_GNT | Minimum grant. |
|  | [15:8] | 08 | Max_LAT | Maximum latency. |
| 0C | [7:0] | 09 | BCR0 | Bus Control Register 0. |
|  | [15:8] | 0E | BCR1 | Bus Control Register 1. |
| 0D | [7:0] | 03 | CFG_A | Configuration register A. |
|  | [15:8] | 00 | CFG_B | Configuration register B. |
| 0E | [7:0] | 40 | CFG_C | Configuration register C. |
|  | [15:8] | 02 | CFG_D | Configuration register D. |
| 0F | [7:0] | 73 | PROG_STATUS | EEPROM programmed status. The value 73H means this EEPROM had been programmed. |
|  | [15:8] | 73 | CHKSUM | EEPROM checksum. Default value is 73H. The checksum is not used at current stage, any value here will be OK. |

NOTE: Devices can work well by using most of the recommended default value. But the values from word offset 00H~05H should be taken care very much, or the device will malfunction. If you don’t know how to program the values from word offset 00H~05H, please contact with the technical supporting engineer.