SMSC Hardware Monitor Software (Windows 2000, XP Version)

Version 1.0 Release Notes

October 17, 2001

Revision History:

Beta Version (September 14, 2001):   Initial Release

Version 1.0 (September 26, 2001):  2nd Release

Changes:	(1) Integrate driver and application installation into one setup.

Compatible Windows Versions:

Microsoft Windows 2000, XP

Tested Chipsets:

- Intel® 815 chipset family with Intel® Chipset Software Installation Utility  Version

v3.20.1008

Introduction:

This Hardware Monitor Software provides hardware monitoring functions for motherboard with SMSC LPC47M192 or 15x series super I/O chips with on-chip hardware monitoring function block.

After the program is activated, a notice window is displayed.

The program will be minimized to the system tray, which is located on the right end of the task bar. The notice window will disappear in 5 seconds.

A popup menu will be displayed by left or right clicking the icon in the system tray.

Select the “Restore HW Monitor” to display the main window; “Unload HW Monitor” to de-activated the program; “Cancel” to cancel the action; or “About” to show version information.

The program consists of two windows: the main window and the setting window. The main window displays voltages, fan speeds and temperature parameters.

All the voltages, temperatures, fan speeds are displayed in this main window in text and slide bar with low and high limits range. If an alarm condition occurs, the main window will pop up (if it is minimized to the system tray) and the corresponding alarm light(s) will be flashing.

By clicking the “Settings” button, a setup window is shown.

In this window, all the low and high limits are set.

Pressing “Tab” key or mouse click in the different fields can change the current editing field. The program will validate the input right after the change of editing focus. If the input is not valid, editing focus will not be changed and the original value is displayed. For example, if the high limit of +5v is just entered but the value is lower that of the low limit, the current editing focus will not change even if the Tab key is pressed. A right number must be entered.

Click “Done” to save the settings. The setting window will be closed and the program returns to the main window.

If “Fahrenheit” is selected in the setting window, the display of temperature in the main window and setting window will be changed right away.

Installation and Uninstall:

(For Windows 2000, before the installation, make sure that the “Intel® Chipset Software Installation Utility  Version v3.20.1008” has been installed in the Windows 2000. “Intel® Chipset Software Installation Utility  Version v3.20.1008” can be downloaded from Intel’s web site.)

The setup file is delivered as one zip file. Unzip the file. Run “Setup.exe” to install.

After installation, the original “Intel 82801BA/BAM SMBus Controller - 2443” will be replaced by “Intel 82801BA/BAM SMBus Controller – 2443 (SMSC Driver)” in “System devices”. An entry of “Hardware Monitor” in the start menu is added and the program can be activated by select the entry from the Windows start menu.

To uninstall the software, use the “Add/remove Programs” in the Control Panel. “HWMonitor” is listed. Highlight it and click “Add/Remove” button to uninstall.

Error Messages:

Driver Load Failure Error – When no SMSC Super I/O chip with hardware monitoring block is

present in the system, an error message box will be displayed to

indicate the failure of loading the driver.

Notes:

The device driver for the SMBus host controller is not ACPI-aware. If there is another driver controlling the SMBus host controller, there will be a conflict between the drivers.

In notebook computers, normally there is an SMBus host controller driver. This version of the SMBus driver does not support notebook computers.