May 16, 1997

YAMAHA Corporation

About PnP ISA Enable Driver

(1)Outline

PnP ISA Enable Driver is being provided by Microsoft for WindowsNT4.0 and OPL3-SA driver corresponds to PnP ISA Enable Driver

(2)Installation of PnP ISA Enable Driver

In WindowsNT4.0 CD-ROM, there are two files which are named PNPISA.SYS and PNPISA.INF in DRVLIB\PNPISA\i386 directory.  Right clicking on PNPISA.INF and choosing “install” will install PnP ISA Enabler.

After restart, PnP ISA Enabler functions will be available.

How to install is also explained in DRVLIB\AUDIO\SBPNP\README.TXT.

(3)Handling and functions of PnP ISA Enable Driver

PnP ISA Enabler detects PnP device which connected to ISA bus at boot and displays a dialog.

(Example)

There are four selections in the dialog as below.

- Windows NT default driver

- Driver from disk provider by hardware manufacture

- Do not install a driver(Windows NT will not prompt you again)

- Select from a list of alternate drivers

Select one of these four and choose OK, or when Cancel was chosen driver was not installed and the device will be detected again next time.

1.Windows NT default driver

This selection is available when a driver which correspondes to PnP ISA Enabler was installed.

Choosing this selection uses pre-installed driver, so it is not required copying files.

2. Driver from disk provider by hardware manufacture

This selection is to install new driver for PnP ISA Enabler and will require an installation disk.

3. Do not install a driver(Windows NT will not prompt you again)

Choose this selection when not installing a driver.  Once this selection was chosen, the device will not be detected next time.  To enable the device to be detected, do the following steps:

- enable PNPISA.SYS

- shut down (power down)

- remove sound board

- start Windows NT

- shut down (power down)

- set sound board

- start Windows NT

This selection is used when the driver dose not correspond to PnP ISA Enabler or Legacy form driver has already been installed.

4. Select from a list of alternate drivers

This selection is to install a driver using information of a driver which corresponds to PnP ISA Enabler on system.  After choosing this selection, a dialog appear to select a hardware type.

Choose an appropriate hardware type

In the case that one of 1,3 or 4 was selected, dialog which is called “Select Device” is appeared.

(Example)

There are two options in the dialog.

- Show compatible devices

- This option lists drivers which correspond to detected device automatically.

- Choose this option in ordinary case.

- Show all devices

- Choose this option when appropriate drivers are not detected by above option.

(4)Environment for installing OPL3-SA series driver.

In WindowsNT4.0, there are two installation ways which are using Control Panel and PnP ISA function.

In WindowsNT3.51, PnP ISA Enable driver is not available so install using Control Panel.

(5)Installing OPL3-SA series driver using PnP ISA function

- Install PnP ISA Enabler.

- Set OPL3-SA series board which corresponds to PnP and restart.

- New Hardware Found dialog which shows OPL3-SAx Sound Board is appeared

- Choose “Driver from disk provider by hardware manufacture” from above dialog and click OK.

- When a dialog which is named “Install from disk” is appeared, appoint the location of directory which contains the driver and click OK.

- Dialog which is called “Select Device” is appeared.

Choose a language which corresponds to the OS and click OK.

7.Configuration dialog is appeared. Click OK.

If there is any change of setting, below dialog will be appeared after clicking OK.

8.Following above steps, GamePort of OPL3-SA series or Reserved Device will be detected

In either case, choose “Do not install a driver(Windows NT will not prompt you again)” and click OK.

If selection was chosen to install driver, OPL3-SAx GamePort or OPL3-SAx Reserved will be appeared as selection.

If the device was selected to install, process was modified to not install and not detect the driver next time, therefore there is no problem with actions.

However, when using “Show all devices” and inappropriate driver was chosen, it is not guaranteed the actions.

- Internal IDE CD-ROM will be detected after all of OPL3-SA series devices are detected.

Choose “Do not install a driver(Windows NT will not prompt you again)” and click OK.

If selection was chosen to install driver, “Standard IDE/ESDI Hard Disk Controller” will be appeared as selection.

If the device was selected to install, process was modified to not install and not detect the driver next time, therefore there is no problem with actions.

However, when using “Show all devices” and inappropriate driver was chosen, it is not guaranteed the actions.

(6)Handling of JoyStick driver

Analog Joystick Driver which is located on WindowsNT4.0 CD-ROM will be available to install by separate installation.

(7)Re-setting

Dialog which corresponds to PnP ISA function will be appeared at Hardware Resource re-setting.  When resource setting is changed manually on this Dialog, after restarting the system this new setting will be available.

(8)Notice

When “Do not install a driver” was chosen for any of OPL3-SAx Sound Board, GamePort, CD-ROM and Internal IDE CD-ROM, the device will not be detected next time.

To enable to be detected again, enable PNPISA.SYS, remove the board and restart, shut down and set the board then restart the system.