Sep 29, 1997

YAMAHA Corporation

The description YAMAHA OPL3-SA Series Driver for WindowsNT

Version4.00.2314 Driver

1. Supported OS and Hardware

1）OS

WindowsNT 4.0/3.51/3.5 (English/ French /German /Italian /Spanish/ Japanese/ U.KEnglish/Danish/Dutch/Finnish/Norwegian/Swedish/Simplified Chinese/Traditional Chinese/Portuguese/Korean)

2）Hardware

YMF701/YMF701B(OPL3SA) /YMF711(OPL3SA2) /YMF715x(OPL3SA3x)

2. Supprted devices

1)Wave-In/Out Device

1.Supported sampling rate :48.0/44.1/37.8/33.075/32.0/27.42/22.05

- 18.9/16.0/11.025/9.6/8.0/6.62/5.51  kHz

2.Format

ADPCM(4-bit IMA compatible)

μ-Law(8-Bit Companded)

A-Law(8-Bit Companded)

Linear(8-Bit unsigned/16-Bit two’s complement, Big Endian)

- Other compressed format is supported by compressed Audio CODEC.

3.FullDuplex

You can use the function if set Use Dual DMA on ON at setting hardware information.

But it is necessary to be consistent with sampling frequency on OPL3-SA2/SA3 board, and with sampling frequency and format(include Stero/Mono) on OPL3-SA board.

2)MIDI-In Device

Utilizable MIDI-In Device:MPU401 midi input(UART Mode)

<Tip>

MPU401 device is valid if set Enable MPU401 on ON at setting hardware

information.

3)MIDI- Out Device

Utilizable MIDI-Out Device have two devices:

FM synthesizer(OPL3)

MPU401 midi output(UART Mode)

<Tip>

MPU401 device is valid if set Enable MPU401 on ON at setting hardware

information.

4)Mixer Device

1.Playback Line

Stereo： Master/Wave/ Synthesizer(FM/external synthesizer) /CD audio/

LineIn/3D Wide

Mono  ： MicIn/MonoIn

<Tip>

a. It is possible to use Mute, but can not use MonoIn/3D Wide

on OPL3-SAx board and 3D Wide on OPL3-SA2 board.

b. It is possible to set On/Off to ZVPort as Advanced Control of Master

on OPL3-SA2 board and set ZVPort/Tone Control on OPL3-SA3x board.

2.Recording Line

Stereo： CD audio/LineIn/Loopback

Mono  ： MicIn

<Tip>

- You can choose each one.

- 3.Other Line

Voice Commands Line : CD audio/LineIn/Loopback (stereo) MicIn (mono)

Recording Monitor

5)Joystick

Joystick is not supported for WindowsNT 3.51.

It is possible to use standard Analog Joystick driver with installing

by other way for WindowsNT 4.0. (Port is only set 201h)

6)DirectSound

- DirectSound is not supported for WindowsNT 3.51.

The function is supported for WindowsNT 4.0, but it is not possible

to use with wave device because OPL3-SA driver is not specially

corresponded to it. It can be used if not operate at same time.

7) OPL3-SAx CONFIGURATION APPLET

1. MIC+20dB

2. ZVPort

<Tip>:

It is valid only on OPL3-SA2.

It is automatic change that ZVPort is disable when FM is played on

OPL3-SA3.

3. ToneControl：Treble/Bass

<Tip>:

- It is valid only on OPL3-SAx.

It is relate with Tone Control of Advanced Control which is

in Master of MixerDevice.

4. 3DEnchanced：Ymersion(level)/mode設定(HiFi/Desktop/NotePC1/NotePC2)

<Tip>:

Ymersion(Level) is valid only OPL3-SA3x.

Ymersion(Level) is relate with 3D Enhanced of Mixer Device.

Mode is valid only on OPL3-SA3B

3. Others

1)APM

Advanced Power Management is not supported.

2)Plug&Play

PnP is not supported for WindowsNT 3.51.

PnP is supported if PNPISA.SYS is installed, and PnPBIOS Information

in registry is used and is corresponded if PNPISA.SYS is not installed

for WindowsNT 4.0. (It is not corresponded to PnP when information about

the series of OPL3-SA is not on PnPBIOS. But driver can set hardware

resource.)

3)Resource

Be sure if there is OPL3-SA resource information in PnPBIOS information

at registry of WindowsNT 4.0. If there is something about OPL3-SA, hardware

resource is only showed in dialog box and can not be changed.(ALL of setting

value is showed by half-tone, just indicate resource. ) If there is nothing

about OPL3-SA, you can set resource as following.

1. WSS(Wave-In/Out,Mixer)

I/O Port: you can choose one of 530h,604h,E80h,F40h.

IRQ     : you can choose one of 3,5,7,9,10,11.

- DMA     : you can choose one of 0,1,3.

<Tip>

a.You can set 608h instead of 604h for I/O Port on OPL3-SA2/SA3

board.

b.IRQ3 and IRQ11 are only valid on OPL3-SA2/SA3 board.

c.You can choose DMA which is one of <Out 0 In 1>,<Out 1 In 0>,

<Out 3 In 1> when use full duplex on OPL3-SA board.

- d.You can set DMA channel of In and Out each when use full duplex

on OPL3-SA2 board or higher.

2. MPU401

I/O Port: you can choose one of 300h,332h,334h,330h.

IRQ     : you can choose one of 3,5,7,9,10,11.

<Tip>

IRQ3 and IRQ11 are valid only on OPL3-SA2/SA3 board.

3. OPL3

I/O Port： only 388h which can not be changed.

Sep 29, 1997

Installation of OPL3-SA Series Driver for WindowsNT

Version4.00.2314

About PnP ISA Enabler Driver(PNPISA.SYS) and OPL3-SAx PnP Driver, see

“About PnP ISA Enabler” (PNPISA.DOC).

The setup disk includes the following files.

Driver for U.S.English/French/German/Italian/Spanish/Japanese

oemsetup.inf

saxpnp.inf

opl3sa32.dll

opl3sa.sys

opl3sax.cpl

\engus\midimap.cfg  (U.S. English)

\fre\midimap.cfg    (French)

\ger\midimap.cfg    (German)

\ita\midimap.cfg     (Italian)

\jpn\midimap.cfg    (Japanese)

- \spa\midimap.cfg    (Spanish)

- Driver for U.S.English/U.K.English/Danish/Dutch/Finnish/Norwegian/Swedish/

- Simplified Chinese/Traditional Chinese/Portuguese/Korean

- oemsetup.inf

- saxpnp.inf

- opl3sa32.dll

- opl3sa.sys

- opl3sax.cpl

- \engus\midimap.cfg  (U.S. English)

- \swe\midimap.cfg   (Swedish)

- \nor\midimap.cfg    (Norwegian)

- \chi_t\midimap.cfg  (Traditional Chinese)

- \chi_t\midimap.cfg  (Simplified Chinese)

- \fin\midimap.cfg    (Finnish)

- \dut\midimap.cfg    (Dutch)

- \dan\midimap.cfg    (Danish)

\enguk\midimap.cfg  (U.K. English)

\por\midimap.cfg    (Portuguese)

\kor\midimap.cfg  (Korean)

1.   How to install the driver

WindowsNT3.51

(1) Start  Microsoft WindowsNT Version3.51.

(2) Logon as Administrator Class user.

- (3) Double click  “Drivers” in the control panel.

- (4) Click “Add” .

- (5) Double click  “Unlisted or Updated Driver” in the list.

- (6) Insert the floppy disk provided by Yamaha.

- (7) Type the drive where the floppy disk is inserted.

- (8) Click “OK”
(9) Select Language of driver in list as following.

- µ §

- (10) Click “OK”

- (11) Set Parameters, 	look next 2.DriverSetup section.

- (12) Shutdown and Restart your Computer.

- WindowsNT4.0

- (1) Start  Microsoft WindowsNT Version4.0.

- (2) Logon as Administrator Class user.

- (3) Double click  “Multimedia” in the control panel.

- (4) Click the “Devices” tab.

- Click “Add” .

- Double click  “Unlisted or Updated Driver” in the list.

- Insert the floppy disk provided by Yamaha.

- (8) Type the drive where the floppy disk is inserted.

- (9) Click “OK”

- (10) Select Language of driver in the list as following.

- µ §

(11) Click “OK”

(12) Set Parameters, 	look next 2.DriverSetup section.

(13) Shutdown and Restart your Computer.

2.   Driver Setup

When you install the driver or  run driver’s setting, the following Driver Configuration Box is shown. (e.g. OPL3-SA3 driver )

µ §

Release note for the OPL3-SA Series WinNT driver

1.Version information and date

supported OS:WinNT ver3.5/3.51/4.0

supported hardware:YMF701/YMF701B(OPL3-SA)/YMF711(OPL3-SA2)/

YMF715x(OPL3SA3x)

Driver for U.S.English/French/German/Italian/Spanish/Japanese

Version: 4.00.2314

File name:opl3sa.sys (9/25/97,101664byte)

opl3sa32.dll (9/25/97,158208yte)

opl3sax.cpl (9/25/97,37376byte)

oemsetup.inf (4/29/97,1339byte)

saxpnp.inf (4/29/97,9135byte)

\engus\midimap.cfg (10/5/96,56418byte)

\fre\midimap.cfg (11/29/96,41606byte)

\ger\midimap.cfg (11/29/96,41606byte)

\ita\midimap.cfg (11/29/96,41606byte)

\jpn\midimap.cfg (11/29/96,41606byte)

\spa\midimap.cfg (11/29/96,42250byte)

Driver for U.S.English/U.K.English/Danish/Dutch/Finnish/Norwegian/Swedish/

Simplified Chinese/Traditional Chinese/Portuguese/Korean

Version: 4.00.2314

File name:opl3sa.sys (9/25/97,105696byte)

opl3sa32.dll (9/25/97,190976byte)

opl3sax.cpl (9/25/97,52224byte)

oemsetup.inf (6/26/97,2115byte)

saxpnp.inf (6/26/97,14613byte)

\engus\midimap.cfg (10/5/96,56418byte)

\swe\midimap.cfg (4/25/96,56418byte)

\nor\midimap.cfg (4/25/96,56418byte)

\chi_t\midimap.cfg (4/21/96,56418byte)

\chi_s\midimap.cfg (4/30/96,56418byte)

\fin\midimap.cfg (4/21/96,56418byte)

\dut\midimap.cfg (4/25/96,56418byte)

\dan\midimap.cfg (4/25/96,56418byte

\enguk\midimap.cfg (4/25/96,56418byte)

\por\midimap.cfg (5/27/97,41606byte)

\Kor\midimap.cfg (6/17/97,41606byte)

# 2.Difference from version the last release

# (1) IN the Config applet, it is now possible to use the arrow keys to operate the Bass, Treble, and Ymersion slide bars.

# (2) We have modified the value that was set to the register relative to the amount of slider change for some mixer. Customization is now possible to choose either Linear settings or operation before.

# 3.Bug list

No known bug.