Chapter 1

Overview

Congratulations on your decision to adopt the PAK-2102 mainboard. The mainboard combines the advanced capabilities of the VIA Apollo 580VP® chipset with a high-performance concurrent PCI local bus architecture to provide the ideal platform for unleashing the unsurpassed speed and power of the Intel Pentium processor. This highly-flexible mainboard is designed to run a full range of Intel Pentium™, Cyrix 6x86™, IBM 6x86™, and AMD-K5™ processors; and can be easily upgraded using its 321-pin ZIF socket. The processor's advanced performance is complemented by a second level write back Pipeline Burst SRAM cache of up to 1MB and main memory of up to 512MB RAM. The main memory is installed using the board's six 72-pin SIMM sockets that accept an unrivaled choice of high-speed EDO and standard Fast Page Mode DRAM.

The PAK-2102 integrates a full set of I/O features onboard, including two 16550 UART compatible serial ports, one EPP/ECP capable parallel port, and one Floppy Disk Drive controller. It also comes with a built-in Enhanced IDE controller that provides convenient, high-speed PCI Bus Master connections with up to four IDE devices, including Hard Disk and CD-ROM drives. An extended slot allows you to install a riser card providing ample room for further  expansion. The mainboard also features support for the state-of-the-art Universal Serial Bus (USB) that provides ease-of-use and high-speed Plug & Play connections to future USB compliant peripheral devices. The IrDA compliant serial port and optional onboard SIR support further enhance system I/O connectivity. Moreover, for advanced VGA functionality, the PAK-2102 features an onboard S3 Trio64V+™ VGA subsystem that delivers up to 1024x768x256 resolutions. The Creative Lab ViBRA 16C™ onboard audio subsystem provides the audio environment.

This chapter gives you a brief overview of the PAK-2102 mainboard. In addition to basic information on the board's main components and features, it also provides advice on how to upgrade and expand it. For updated BIOS, drivers, or product release information, please visit FIC's home page at: http://www.fic.com.tw.

# Main Features

The PAK-2102 mainboard comes with the following high-performance features:

- Easy Installation

- Phoenix BIOS with support for Plug and Play, auto detection of Hard Drive

- and IDE features, and MS Windows 95™.

- Flexible Processor Support

- The onboard 321-pin ZIF socket supports Intel Pentium (P54C) CPU speed

75/90/100/120/133/150/166/200 MHz processors / P54CS / P54CTB / P55.P55C.

.Cyrix 6x86-P120+ (100 MHz) / 6x86-P133+ (110 MHz) / 6x86-P150+

(120 MHz) / 6x86-P166+ (133 MHz) / M2 seris processors.

IBM 6x86-P120+ (100 MHz) / 6x86-P133+ (110 MHz) / 6x86-P150+

(120 MHz) / 6x86-P166+ (133 MHz) / M2 seris processors.

A...AMD K5-PR75 (75 MHz) / K5-PR90 (90 MHz) / K5-PR100 (100 MHz) /

K5-PR120 (90 MHz) / K5-PR133 (100 MHz) / K5-PR150 (105 MHz) /

K5-PR166 (116 MHz) / K5-PR200 (133 MHz) processors.

- Leading Edge Chipset

- VIA 580VP chipset, including a CPU interface controller, advanced

- cache controller, integrated DRAM controller, synchronous ISA bus

- controller, PCI local bus interface, integrated power management unit.

- Ultra-fast Level II Cache

Supports onboard Pipeline Burst SRAM that can be expanded to 1MB

using a cache module.

- Versatile Main Memory Support

- Accepts up to 512MB RAM in threebanks using six SIMMs of 4, 8, 16, 32, ||     32, 64, 128MB with support for FPM and EDO DRAM.

- ISA & PCI Expansion Slots

- An extend slot allows a riser card for further expansion.

- USB Support (Reserved for Future Upgrade)

- Two Unniversal Serial Bus plug-in connectors integrated into rear I/O

- panel.

- Enhanced PCI Bus Master IDE Controller

- Integrated Enhanced PCI local bus IDE controller with two connectors

- supports up to four IDE devices such as Hard Disk, CD-ROM or Tape

Backup drives via two channels for high speed data throughput. This

controller supports PIO Modes 3 and 4, and DMA Mode 2 for optimized

system performance.

- Super Multi I/O

- Integrated NS PC87336VLJ Multi I/O chipset features two 16550A UART comcompatible serial  ports, one EPP/ECP capable parallel port, one IR port, an  a|  nand one Floppy Disk  Drive connector.

- Compact Audio Onboard Subsystem

Integrated Creative Lab ViBRA 16C onboard audio system controller and

Yamaha OPL ¾ FM music synthesizer fully compatible with Sound BlasBlaster®, Sound Blaster Pro, Microsoft Windows Sound system, and Rol||Roland MPU401 standard.

- High-Resolution Onboard VGA Subsystem

||Onboard S3 Trio64V+ VGA subsystem offers up to 1280x1024x256 ||||||||||resolution.

This Page Intentionally Left Blank