Chapter 1

Overview

Based on the new highly-integrated Intel 430TX PCIset,  the PT-2012 combines blistering PentiumÒ processor performance with support for intelligent diagnostic and power management features like Hardware Monitoring, DMI (Desktop Management Interface) and ACPI (Advanced Configuration and Power Interface), to provide a powerful and versatile ATX-size platform for leading-edge PC ’97 compliant systems.

With its switching voltage regulator, the PT-2012 runs a complete range of Intel PentiumÒ processors, including the Intel Pentium processor with MMXÔ technology, as well as the AMD-K5Ô and Cyrix/IBM 6x86Ô, and is easily upgradable to the Cyrix/IBM MXÔ and the AMD-K6Ô. For added power and performance, the PT-2012 takes up to 512KB Pipeline Burst Level II cache and up to 256MB DRAM via three 168-pin DIMM sockets which accept high-speed  3.3 V EDO DRAM and lightning-fast SDRAM memory types.

The PT-2012 comes with a full set of I/O features conveniently integrated on the rear I/O panel, including two USB connectors. The board also has an integrated PCI Bus Master Enhanced IDE controller with support for the new Ultra DMA/33 protocol, which doubles ATA-2 Hard Disk Drive data transfer rates to 33MB/s while maintaining full backwards compatibility with existing PIO Mode 3, PIO Mode 4 and DMA Mode 2 devices.

Fully compliant with the Microsoft PC’97 standard at both the hardware and BIOS levels, the PT-2012 comes with support for intelligent Hardware Monitoring and DMI features which continuously check the thermal status of your system and reduce the cost of ownership through improved manageability. The mainboard also comes with the feature of LANDesk Client Manager (LDCM) that uses the DMI standard established by the Desktop Management Task (DMTF) so that the manageable components can be viewed across multiple platforms.

For the most up-to-date information about your mainboard and the latest FAQs and BIOS updates, visit FIC Online at www.fic.com.tw.

# Main Features

The PT-2012 mainboard comes with the following high-performance features:

- Easy Installation

Award BIOS with support for auto-detection of Hard Disk Drives, Plug and Play devices, and PS/2 keyboard and mouse, to facilitate the installation of HDDs, expansion cards and other peripheral devices.

- Leading Edge Chipset

Intel 82430TX PCIset, a two-chip BGA solution with integrated DRAM and LII cache controllers as well as support for Intel's new Dynamic Power Management Architecture (DPMA), Concurrent PCI (PCI 2.0 and 2.1), and USB.

- Flexible Processor Support

Onboard 321-pin ZIF socket and switching voltage regulator support complete range of leading-edge processors:

Intel PentiumÒ MMXÔ technology 166/200/233 MHz processors.

Intel PentiumÒ P54C/P54CS 90/100/120/133/150/166/200 MHz processors.

|||||||||AMD-K6Ô-166 (166 MHz) / K6-200 (200 MHz) / K6-233 (233 MHz)    |||||||| processors.

|||||||||AMD-K5Ô- PR90 (90 MHz) / K5-PR100 (100 MHz) / K5-PR120 (90     |||||||||MHz) / K5-PR133 (100 MHz) / K5-PR150 (105 MHz) / K5-PR166 (116 |||||||||MHz) / K5-PR200 (133 MHz) processors.

Cyrix 6x86MXÔ- PR166 (150 MHz) / 6x86MX-PR200 (166 MHz) / 6x86- |||||||||MX-PR233 (200 MHz) / 6x86MX-PR266 (233 MHz) processors.

|| || | Cyrix 6x86Ô- PR133+ (110 MHz) / 6x86-PR150+ (120 MHz) / 6x86-|||||| |||||||||PR166+ (133 MHz) processors.

IBM 6x86MXÔ- PR166 (150 MHz) / 6x86MX-PR200 (166 MHz) / 6x86- |||||||||MX-PR233 (200 MHz) / 6x86MX-PR266 (233 MHz) processors.

||  |   IBM 6x86Ô- PR133+ (110 MHz) / 6x86-PR150+ (120 MHz) / 6x86- |||||||||PR166+ (133 MHz) processors.

- Various External Bus and CPU/Bus Frequency Ratio Support

The mainboard supports the Bus frequency of 55 / 60 / 66 MHz and the CPU/Bus frequency ratio of 1x / 1.5x / 1.75x / 2x / 2.5x / 3x / 3.5x / 4x.

|||||||||(Please refer to Sec. Install the CPU in Chapter 2 for more information).

- Ultra-fast Level II Cache

Supports 512KB onboard Pipeline Burst Level II direct-mapped write-back cache.

- Versatile Main Memory Support

Accepts up to 256MB using three DIMMs of 8, 16, 32, 64, 128MB with support for and 3.3V EDO DRAM and lightning-fast SDRAM.

- ISA & PCI Expansion Slots

Four 16-bit ISA and four 32-bit PCI expansion slots provide all the room you need to install a full range of add-on cards.

- Enhanced PCI Bus Master IDE Controller with Ultra DMA/33 Support

Integrated Enhanced PCI Bus Master IDE controller features two dual-channel connectors that accept up to four Enhanced IDE devices, including CD-ROM and Tape Backup Drives, as well as Hard Disk Drives supporting the new Ultra DMA/33 protocol which doubles data transfer rates to 33MB/sec. Standard PIO Mode 3, PIO Mode 4, and DMA Mode 2 devices are also supported.

- Super Multi I/O

Integrated NS PC87309 Plug and Play multi-I/O chipset features two high-speed 16550A compatible serial ports, one EPP/ECP capable parallel port, and one FDD connector.

- USB Support

Two USB ports integrated in the rear I/O panel allow convenient, high-speed Plug and Play connections to the growing number of USB compliant external peripheral devices on the market.

- Optional IrDA Connector

An optional IrDA connector for wireless infrared connections is available.

- LDCM Support

The onboard connector supports PCI LAN cards that equipped AMD PCnet-ISA IIÔ (79C961A) and PCnet-PCI IIÔ (79C9790A) chipsets for allowing remote waking-up (PT-2012 as a workstation) and supports LDCM 3.0 feature.

This Page Intentonally Left Blank