FIC PN-6210 PCI SYSTEM BOARD

BOARD LEVEL COMPATIBILITY TEST REPORT

23 September 1996

Document Number:  	MB-275

Product Version:  		V 1.2

BIOS Version:  		N11-100

| Supervisor | Writer |  |
| --- | --- | --- |
| Mick Yen |  | Tony Chou |

FIC Research & Development Department

5F #52 Min-Chuan Road, Hsin-Tien City Taipei, Taiwan, R.O.C

Tel : 886-2-918-6001     Fax : 886-2-915-6728

TABLE OF CONTENTS

- Introduction

- Mainboard Features

- Mainboard CPU Support

- System Specification for Hardware and Software Utilities

- Jumper Function Verification

- Field (Stress ) Test : Dynamic Burn-in Tests

- Base Operating System SCT/HCT Tests

- Hardware Compatibility Test Suite

  - Definition of Compatibility

  - Display adapter Products

  - SCSI Adapter Products

  - SCSI Peripheral Products

  - IDE Fixed Disk Drive Products

  - Floppy Disk Drive Products

  - Network card Products

  - ATAPI CD-ROM Device Products

  - Printer Peripheral Products

  - Mouse Products

  - Sound Blaster Products

  - MPEG Video Products

  - FAX / Modem Products

  - Power Supply Products

  - DRAM Module

- Software Compatibility Test Suite

  - Base Operating System

  - Software tested list 
Suites, Word Processing, Spreadsheets, Presentation Programs, CAD, Communication, Databases, Graphics, Programs, Diagnostic Programs, Tools, Time Management, Scanners CD Titles, CD Games, CD Titles (Multimedia)

# Introduction

FIC’s motherboards undergo a comprehensive suite of stringent software and hardware compatibility tests to ensure that they can not only run a complete range of operating systems and software packages, but that they can also be installed with a full selection of add-on cards and peripheral devices, such as VGA, SCSI, and network adapters as well as HDDs, FDDs, CD-ROM Drives and Fax/Modems. A board is deemed to have passed the compatibility testing suite when it has met the following criteria:

- All application software available for IBM compatible products works properly.

- All common internal and external hardware available to the IBM AT-Bus and PCI compatible systems functions properly.

- All hardware and software can be installed according to the instructions provided to the  user.

- All legal configurations (IRQ , I/O addresses, DMA levels ) function properly.

- All applications have been tested with and without drivers provided on the support disk.

- All operating systems have been installed and tested with each processor, with and without  the most critical mode selected by setup.

# Mainboard Features

Model Name :	PN-6210 V1.2

Product Name :	Single and Dual Pentium Pro-Processor  Based Mainboard

System BIOS :	AMI BIOS Version: NII 1.00

PCISet :	Intel Natoma PCI Chipset (82440FX)

PCI IDE :	Intel PIIX3 Dual Bus Master PCI IDE

I/O :	NS 336 support EPP and ECP mode and IR Interface

# Mainboard CPU Support:

| Intel CPUs | Status | Intel CPUs | Status |
| --- | --- | --- | --- |
| Single Pentium Pro 150 | Pass | Dual Pentium Pro 150 | Pass |
| Single Pentium Pro 166 | Pass | Dual Pentium Pro 166 | Pass |
| Single Pentium Pro 180 | Pass | Dual Pentium Pro 180 | Pass |
| Single Pentium Pro 200 | Pass | Dual Pentium Pro 200 | Pass |

# System Specification for Hardware and Software Utilities

The mainboard hardware portion was configured as follows :

| Item | Description |
| --- | --- |
| System Core Logic | Intel 440FX (Natoma) PCI set (B0) |
| System Memory | Maximum : 256 MB (Two Bank) |
|  | 72 pins single RAS and Double RAS RAM Module |
|  | Fast Page / EDO Memory Supports |
| PCI Bus clock | Synchronous mode (depended on CPU Clock divide by 2) |
| System Slots | 4 PCI and 3 ISA slots |
| I/O Core Logic | NS 336 (EPP/ECP and IR support) |
| PCI IDE | PIIX3 Dual Bus Master PCI IDE |

# Jumper Function Verification

| Description | Jumper Name | Default Option | Status |
| --- | --- | --- | --- |
| Speaker Connector | SPK | Open | Normal |
| Keyboard Lock and Power LED | KB_Lock | Open | Normal |
| Turbo LED Connector | TB_LED | Open | Normal |
| IDE LED Connector | IDE_LED | Open | Normal |
| Remote Power Switch | RPW_SW | Open | Normal |
| Suspend Switch | SP_SW | Open | Normal |
| Suspend LED | SP_LED | Open | Normal |
| Reset Switch | RST | Open | Normal |
| CPU Clock Rate | FREQ1/FEQ2/FREQ3/FREQ4 | Open/Short | Normal |
| CPU Clock Selector | CLK1/CLK2 | Open/Short | Normal |
| CPU FAN Connector | P_FAN / S_FAN | Open | Normal |
| Clear Password Switch | CPS | Open/Short | Normal |
| ROM Type Select | EP1/EP2 | 1-2/2-3 | Normal |

# Field (Stress) Test : Dynamic Burn-in Tests

| Software Name | Duration | Result |
| --- | --- | --- |
| BAPCO 95 for Windows NT | 24 hr. | Pass |
| Test Configurations for NT | Intel Pentium Pro 200-MHz FIC 1st Grafx-65MP 1MB PCI VGA Card Adaptec AHA-2940UW ST-32550 SCSI-2 On Board 64 MB Memory |  |
| Novell Server version 4.1 | 48 hr. | Pass |
| Test Configurations for Novell | Intel Pentium Pro 200 MHz Matrox Millennium 2MB WRAM PCI VGA Card Adaptec AHA-3940 2 set Quantum-540S SCSI (Dual channel Disk mirror) 3COM 3C595 10/100 PCI Netware card (Connect to 8~10 workstation ) D-Link DE530 Ethernet card (Connect to 4 work station On Board 32MB Memory Test Program : Novell Nwtest. |  |
| SCO Unix Open Desktop 5.0 | 24 hr. | Pass |
| Test Configurations for SCO | Intel Pentium Pro 200MHz FIC 1st Grafx-65MP 2MB EDO PCI VGA Card Goldstar GCD-540B APAPI 4X CD-ROM Conner 1.6GB CFS-1621A Novell NE2000 Plus (Connect 3 DOS TCP/IP Workstation) On Board 32MB Memory Test Program : Create a batch |  |

| Software Name | Duration | Result |
| --- | --- | --- |
| IBM OS/2 Warp 3.0 | 24 hr. | Pass |
| Test Configurations for OS/2 | Intel Pentium Pro 200 MHz FIC 1st Grafx-65MP 2MB EDO PCI VGA Card Panasonic CDR-581 4x CD-ROM Seagate 1GB ST-1080A On Board 32MB Memory Test Program : Create a batch |  |
| Winstone 96 Under WIN95 | 48hr. | Pass |
| Test Configurations for Winstone 96 | Intel Pentium Pro -200 MHz Matrox Millennium 2MB WRAM PCI VGA Goldstar GCD-580B 8x CD-ROM Quantum 1.2GB FB-1280A On Board 32MB Memory Tests condition : Run continuos demo mode ( Must skip dbase ). |  |
| BACPO 95 for WFW V3.11 | 24hr. | Pass |
| Test Configurations for BACPO 95 under Windows for Workgroups V3.11 | Intel Pentium Pro 200 MHz Diamond Viper stealth 64 4MB VRAM PCI VGA card Quantum 1.2GB FB-1280A On Board 32MB Memory Tests condition : Run continuous demo mode ( Must skip dbase ). |  |

# Base Operating Systems  Certification Tests

| Operating System | Company | HCT / SCT Name | Result |
| --- | --- | --- | --- |
| IBM OS/2 Version 3.0 | IBM | OS/2 SCT 3.0 | Pass |
| Test Configurations | Intel Pentium Pro -200 MHz TEAC 1.44 MB FDD Seagate ST-31220A On Board 32MB Memory |  |  |
| Windows NT Version 3.51 | Microsoft | Window NT SCT 3.51 | Pass |
| Test Configurations | Intel Pentium Pro -200 MHz YEDA -702J Adaptec AHA-2940UW Quantum FB-1280S 1.2GB SCSI HDF On Board 32MB Memory |  |  |
| Windows 95 | Microsoft | Window 95 SCT 4.0a | Pass |
| Test Configurations | Intel Pentium Pro -200 MHz TEAC 1.44 MB FDD Quantum FB-1280A 1.2GB IDE HDD NEC CDR-1300 ATAPI 6x CD-ROM On Board 32MB Memory |  |  |

# Hardware Compatibility Test Suite

## Definition of Compatibility

A set of hardware and software is defined to demonstrate an acceptable level of compatibility when :

- All application software available for IBM compatible products works properly.

- All common internal and external hardware available to the IBM AT-Bus and PCI compatible system function properly.

- All hardware and software can be installed according to the instructions provided to the user.

- All legal configurations (IRQ , I/O addresses, DMA levels ) function properly.

- All applications  have been tested with and with out drivers provided on the support disk.

- All operating systems have been installed and tested with each processor, with and with out the most critical mode selected by setup.

## Display Adapter Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| ATI | Mach 64 | Graphic Pro-Turbo 2MB | PCI | Pass |
|  | Mach 64 VT | Video Xpression 2MB 3D | PCI | Pass |
| Alliance | ProMotion 6422 | Alliance Pro Motion 6422 | PCI | Pass |
| Diamond | Stealth 64 VRAM | Stealth 64 2/4 MB VRAM (S3 968) | PCI | Pass |
|  | Stealth 64 | Stealth 64 PCI VGA Card (S3 964) | PCI | Pass |
|  | Stealth 64 DRAM | Stealth 64 2MB DRAM (S3 864) | PCI | Pass |
|  | Viper SE | Viper SE Weitek P9100 | PCI | Pass |
|  | Edge 3D 2120XL | Edge 3D 2120XL 2MB DRAM | PCI | Pass |
|  | Edge 3D 3240XL | Edge 3D 3240XL 2MB VRAM | PCI | Pass |
| FIC | Trio 32P | S3 732 1/2 MB DRAM PCI VGA | PCI | Pass |
|  | 1stGrafx- 64P | S3 764 1/2 MB DRAM | PCI | Pass |
|  | 1st Grafx-65MP | S3 765 1/2 MB DRAM | PCI | Pass |
|  | 1st Grafx-67MP | S3 ViRGE 1/2 MB EDO DRAM | PCI | Pass |
|  | 1st Grafx-40MP | Cirrus 54M40 1/2 MB DRAM | PCI | Pass |
|  | 1st Grafx-46MP | Cirrus 5446 1/2 MB DRAM | PCI | Pass |
| Leadtek | WinFast-S280 | S3 765 1/2 MB EDO DRAM | PCI | Pass |
|  | WinFast-3D-S600 | S3 ViRGE 1/2 MB EDO DRAM | PCI | Pass |
| Matrox | MAG-PCI/2 | Matrox PCI 2MB VRAM | PCI | Pass |
|  | Millennium 3D | Matrox 4MB VRAM | PCI | Pass |
| Number 9 | Number 9 | #9 GXE 64Pro | PCI | Pass |
| Trident | 9680 | Trident 9680 2M DRAM | PCI | Pass |

## SCSI Adapter Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| Adaptec | AHA-1530P | Adaptec AHA-1530P ISA PnP SCSI | ISA PnP | Pass |
|  | AHA-1542 | Adaptec AHA-1542C ISA SCSI | ISA | Pass |
|  | AHA-2940 | Adaptec AHA-2940 PCI SCSI card | PCI | Pass |
|  | AHA-2940UW | Adaptec AHA-2940UW PCI SCSI card | PCI | Pass |
|  | AHA-3940 | Adaptec AHA-3940 PCI to PCI Bridge card | PCI | Pass |
|  | AHA-3985 | Adaptec AHA-3985 PCI SCSI card | PCI | Pass |
| Buslogic | BT-946 | Buslogic BT-946 SCSI | PCI | Pass |
|  | BT-948 | Buslogic BT-948 SCSI | PCI | Pass |
|  | FlashPoint LT | Buslogic BA81C15 PCI SCSI | PCI | Pass |
| Feature | PnP 1630 | Feature Domain 9441 PnP ISA SCSI | ISA PnP | Pass |
| Domain | TMC-3260 | Feature Domain 36C70 PCI SCSI | PCI | Pass |
| FIC | SCSI-810P | NCR-810P PCI SCSI Card | PCI | Pass |
| NCR | 825S | NCR-825S PCI SCSI Card | PCI | Pass |
| Symbios | 875 | Symbios-875 PCI SCSI Card | PCI | Pass |
| Initio | INI-9100HA/W | Initio Ini-9100HA/W PCI SCSI | PCI | Pass |

## SCSI Peripheral Products

| ITEM | Vendor | Model Name | Product Description | Result |
| --- | --- | --- | --- | --- |
| Hard Disk | Conner | CFA-540S | 540MB SCSI Hard Disk Drive | Pass |
|  | Quantum | LPS-540S | 540MB SCSI Hard Disk Drive | Pass |
|  |  | FB-1080S | 1GB SCSI Hard Disk Drive | Pass |
|  | Seagate | ST-32550W | 2GB SCSI Hard Disk Drive | Pass |
| CD-ROM | SONY | CDU-55S | SCSI CD-ROM Drive 2X | Pass |
|  |  | CDU-76E | SCSI CD-ROM Drive 4X | Pass |
|  | NEC | CDR-222 | SCSI CD-ROM Drive 2X | Pass |
|  |  | CDR-510 | SCSI CD-ROM Drive 4X | Pass |
| TAPE | Tandberg | TDC 3820 | 525MB SCSI Tape Drive | Pass |
|  | Wangtek | 5525ES | 525MB SCSI Tape Drive | Pass |
| DAT | Wangtek |  | SCSI DAT 2.0G | Pass |
| Iomega | Ditto | IO80Fi | Ditto Driver 800 Tape Insider | Pass |
|  |  | IO80-PX3 | Ditto Easy 800 External Tape Driver | Pass |
|  | Zip | Z100i | Zip 100 SCSI Insider | Pass |
|  |  | Z100S | Zip 100 SCSI | Pass |

## IDE Fixed Disk Drive Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| Quantum | BF-1280 | 5.25” 1.2GB PIO mode 4 / DMA mode 1 | IDE | Pass |
|  | BF-2550 | 5.25” 2.5GB PIO mode 4/ DMA mode 1 | IDE | Pass |
| Conner | CFA-850A | 850MB PIO mode 4 / DMA mode 1 | IDE | Pass |
|  | CFS-1080A | 1GB PIO mode 4 / DMA mode 1 | IDE | Pass |
|  | CFS-1621A | 1.6GB PIO mode 4/ DMA mode 1 | IDE | Pass |
| Fujitsu | M2684TA | 540MB PIO mode 3 / DMA mode 0 | IDE | Pass |
|  | M3684TA | 850MB PIO mode 3 / DMA mode 1 | IDE | Pass |
| IBM | DPEA-31080 | 1GB PIO mode 3 / DMA Mode 0 | IDE | Pass |
|  | DJAA-21080 | 1.7 GB PIO Mode 4 / DMA Mode 1 | IDE | Pass |
| Maxtor | 712626AP | 1.6GB PIO mode 3 / DMA mode 1 | IDE | Pass |
| Seagate | ST-3660A | 540MB PIO mode 3 / DMA mode 0 | IDE | Pass |
|  | ST-850A | 850MB PIO mode 4 / DMA mode 1 | IDE | Pass |
|  | ST-31220A | 1.2GB PIO mode 4 / DMA mode 1 | IDE | Pass |
|  | ST-51270A | 1.2GB PIO mode 4 / DMA mode 1 | IDE | Pass |
| Quantum | FB-2550 | 1GB PIO mode 4 / DMA mode 1 | IDE | Pass |
|  | FB-1280A | 1.2GB PIO mode 4 / DMA mode 1 | IDE | Pass |
| WDC | AC2540 | 540MB PIO mode 3 / DMA mode 1 | IDE | Pass |
|  | AC21000 | 1GB PIO mode 4 / DMA mode 1 | IDE | Pass |
|  | AC21200 | 1.2GB PIO mode 4 / DMA mode 1 | IDE | Pass |
|  | AC31600 | 1.6GB PIO mode 4 / DMA mode 1 | IDE | Pass |

## Floppy Disk Drive Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| EPSON | SMD-1340-301 | 1.2MB Floppy Disk Drive | FDC | Pass |
|  | SMD-1304-301 | 1.44 MB Floppy Disk Drive | FDC | Pass |
| Mitsumi | D509V3 | 1.2MB Floppy Disk Drive | FDC | Pass |
|  | D35375 | 1.44 MB Floppy 3 Mode Disk Drive | FDC | Pass |
| Panasonic | JU-475-3AG1 | 1.2MB Floppy Disk Drive | FDC | Pass |
|  | JU-257 | 1.44 MB Floppy Disk Drive | FDC | Pass |
|  | JU-259 | 2.88 Floppy Disk Drive | FDC | Pass |
| Sony | MPF-520-1 | 1.44MB Floppy Disk Drive | FDC | Pass |
| TEAC | FD-55GF | 1.2 MB Floppy Disk Drive | FDC | Pass |
|  | FD-235HG | 1.44 MB Floppy Disk Drive | FDC | Pass |
|  | FD-235J | 2.88 Floppy Disk Drive | FDC | Pass |
|  | FD-235HF | 3 Mode MB Floppy Disk Drive | FDC | Pass |
| YEDATA | YD-380 | 1.2MB Floppy Disk Drive | FDC | Pass |
|  | YD-702J | 1.44 MB Floppy Disk Drive | FDC | Pass |

## Network Card Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| 3COM | 3C905 | 3COM Etherlink III 10/100 PCI | PCI | Pass |
|  | 3C509 | 3COM Etherlink III | ISA | Pass |
|  | Combo 3C509B | 3COM Ethernet PnP ISA Network card | ISA PnP | Pass |
|  | 3C-590 | 3COM Etherlink III PCI | PCI | Pass |
|  | 3C-5950TX | 3COM Etherlink III 10/100 PCI | PCI | Pass |
| Accton | ACC40 | Accton ACC40 Ethernet card | ISA | Pass |
|  | EN-166X | PnP ISA Ethernet card | ISA PnP | Pass |
|  | PCI-40 | Digital DC1003C PCI Network | PCI | Pass |
|  | EN1207-TX | Accton 100Mbps Ethernet card | PCI | Pass |
| C-NET | CN970EBT | AMD AM97C970-A4 PCI Ethernet | PCI | Pass |
|  | CN100TX | Digital DC1010BA 10/100 Mbps Ethernet Card | PCI | Pass |
| Intel | Ether Express Pro/100 | Intel Ether Express Pro/100 LAN Adapter | PCI | Pass |
|  | Express 16 | Intel Express 16 ISA LAN Adapter | ISA | Pass |
| IBM | IBM 04H8095 | IBM 73G2692 Auto LAN Streamer PCI | PCI | Pass |
|  | IBM 92G7632 | IBM 23H3004 Auto 16/4 Token-Ring | ISA PnP | Pass |
| Olicom | OC-3136 | Olicom PCI Token-Ring card | PCI | Pass |
|  | OC-3118 | Olicom ISA PnP Token-Ring card | ISA PnP | Pass |
| SMC | SMC 8432BT | SMC Ether Power PCI Combo | PCI | Pass |
|  | SMC9332DST | SMC Ether Power 10/100 PCI | PCI | Pass |
|  | EtherEZ | SMC Ethernet Card | ISA PnP | Pass |
| ZNXY | ZX-312 | Digital DC-1003 PCI Ethernet | PCI | Pass |
| D-Link | DE-530 CT | Digital PCI Ethernet card | PCI | Pass |
|  | EZ-2000CT | EZ2000+ Series ISA Ethernet card | ISA | Pass |
|  | EZ-220 CT | PnP ISA Ethernet card | ISA PnP | Pass |
| Novell | NE2000 Plus | Novell NE2000 Plus | ISA | Pass |
|  | NE2100 | Novell NE2100 | ISA | Pass |
| Thomas | TC-4045 | Thomas Corned Token-Ring | ISA | Pass |
| Corned | TC-4145 | Thomas Corned PnP ISA Token-Ring | ISA PnP | Pass |
| Olicom | OC-3118 | Olicom ISA PnP Token-Ring Card | ISA PnP | Pass |

## ATAPI CD-ROM Device Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| AZTECH | CDA668-01I | ATAPI CD-ROM Device 6X | IDE | Pass |
| Aelta | ODC-6101 | ATAPI CD-ROM Device 6X | IDE | Pass |
| Acer | 665A-002 | ATAPI CD-ROM Device 6X | IDE | Pass |
| Creative | MK-4013 | ATAPI CD-ROM Device 4X | IDE | Pass |
|  | MK-4014 | ATAPI CD-ROM Device 6X | IDE | Pass |
| GoldStar | GCD-R540 | ATAPI CD-ROM Device 4X | IDE | Pass |
|  | GCD-R542B | ATAPI CD-ROM Device 4X | IDE | Pass |
|  | GCD-R580B | ATAPI CD-ROM Device 8X | IDE | Pass |
| Hitachi | CDR-7730 | ATAPI CD-ROM Device 4X | IDE | Pass |
|  | CDR-7830 | ATAPI CD-ROM Device 6X | IDE | Pass |
| NEC | CDR-273 | ATAPI CD-ROM Device 4X | IDE | Pass |
|  | CDR-1300A | ATAPI CD-ROM Device 6X | IDE | Pass |
| Panasonic | CDR-581-B | ATAPI CD-ROM Device 4X | IDE | Pass |
|  | CDR-581-M | ATAPI CD-ROM Device 4X | IDE | Pass |
| Sony | CDU55E | ATAPI CD-ROM Device 2X | IDE | Pass |
|  | CDU311-10 | ATAPI CD-ROM Device 8X | IDE | Pass |
| Samsung | SCR-830 | ATAPI CD-ROM Device 8X | IDE | Pass |
| TATUNG | CD-650B | ATAPI CD-ROM Device 6X | IDE | Pass |
| TEAC | CD-561 | ATAPI CD-ROM Device 6X | IDE | Pass |
| Torisan | CDR-S18 | ATAPI CD-ROM Device 8X | IDE | Pass |
| Vertos | 600ESD | ATAPI CD-ROM Device 6X | IDE | Pass |
| Wearnes | CD-220 | ATAPI CD-ROM Device 4X | IDE | Pass |
|  | CD-620 | ATAPI CD-ROM Device 6X | IDE | Pass |
|  | CDD-820 | ATAPI CD-ROM Device 8X | IDE | Pass |

## Printer Peripheral Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| HP | Laserjet III | HP Laserjet III | LPT | Pass |
|  | Laserjet 5P | HP Laserjet 5P support IR Interface | LPT | Pass |
| D-Link | DE-EPP | D-Link EPP Pocket LAN | LPT | Pass |
| Up-select |  | External EPP SCSI BOX | LPT | Pass |
| Road | Express T305 | External Parallel Port Tape Backup | LPT | Pass |
| Runner | Express FD144 | External Parallel Port Floppy Drive | LPT | Pass |
|  | Express CD-4X | External Parallel Port CD-ROM Driver | LPT | Pass |
|  | Express XD | External Parallel Port Hard Driver | LPT | Pass |
| Actisys | ACT-IR210L | InfraRed Computer Link | LPT | Pass |

## Mouse Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| Microsoft | Microsoft Mouse | Microsoft Serial - PS/2 Mouse | KB / Serial | Pass |
|  | Microsoft Mouse | Mouse Port compatible Mouse 2.0A | Keyboard | Pass |
| Logitech | M-S34-6MD | PS/2 compatible Mouse | Keyboard | Pass |
|  | S3-34 | Serial Mouse | Serial | Pass |
| Qtronix | Lynx-25P | PS/2 Compatible Mouse | Keyboard | Pass |
|  | Lynx-25S | Serial Mouse | Serial | Pass |
|  | Easy | Serial Mouse 3-Button | Serial | Pass |
|  | Kwik-200/PS2 | PS/2 compatible Mouse | Keyboard | Pass |
|  | Kwik-200/Serial | Serial Mouse | Serial | Pass |
|  | M-30P | PS/2 compatible Mouse | Keyboard | Pass |
| Actisys | ACT-IR22--L | InfraRed Computer Link | Serial | Pass |

## Sound Blaster Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| Avance | ALS-100 | Avance Logic PnP Sound | ISA PnP | Pass |
| AZTECH | Nova 16 Extea II | Aztech Sound card | ISA | Pass |
|  | Waverider 32+ | Aztech Sound Galaxy Waverider 32+ | ISA | Pass |
|  | AZT2320 | Aztech Waverider32 + | ISA PnP | Pass |
| AMD | InterWave | AMD AM78C201KC PnP Sound | ISA PnP | Pass |
| Diamond | Multimedia Kit 6000 | Diamond Viper Multimedia Kit 6000 | ISA | Pass |
| JAZZ | Sound card | Jazz sound card | ISA | Pass |
| AMD | Inter Wave | AMD AM78C201KC PnP Sound | ISA PnP | Pass |
| Creative | AWE32 | Creative AWE32 Sound Blaster | ISA PnP | Pass |
|  | CT-2890 | Creative ISA PnP Sound Blaster | ISA PnP | Pass |
|  | CT-2940 | Creative ISA PnP Sound Blaster | ISA PnP | Pass |
|  | CT-2950 | Creative ISA PnP Sound Blaster | ISA PnP | Pass |
|  | CT-3600 | Creative ISA PnP Sound Blaster | ISA PnP | Pass |
|  | CT-3792 | Creative ISA PnP Sound Blaster | ISA PnP | Pass |
|  | CT-3990 | Creative ISA PnP Sound Blaster | ISA PnP | Pass |
| Everex | Wave Machine | Crystal CS4332 PnP Sound card | ISA PnP | Pass |
| ESS | ESS-1688 | ESS ISA PnP sound Blaster | ISA PnP | Pass |
|  | ESS-1868 | ESS ISA PnP sound Blaster | ISA PnP | Pass |

## MPEG Video Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| FIC | 1stGrafx-CP3 | S3 443 1/2 DRAM | PCI | Pass |
| Creative | Video Blaster SE-100 | Creative SE-100 MPEG card | ISA | Pass |
|  | Video Blaster RT300 | Creative RT-300 Video Capture board | ISA | Pass |
| Formosa | MPEG-MP020 | Formosa Full Function MPEG card | ISA | Pass |
|  | MPEG-MP021 | Formosa Full screen MPEG | ISA | Pass |
|  | TV-Tuner | Formosa TV-Tuner board | ISA | Pass |
| Real Magic | MPEG | Real Magic Full Function MPEG card | ISA | Pass |

## FAX / Modem Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| Creative | WS-2814EM2 | Creative Modem Blaster 28.8 External | External | Pass |
|  | Phone Blaster | Creative Phone Blaster | Internal | Pass |
| Diamond | Telecommander | Diamond Telecommander 3500XL | Internal | Pass |
| HAYES | ACCURA | HAYES ACCURA 288V.FC + FAX | Internal | Pass |
| Megahertz |  | Internal 2.88 Modem | Internal | Pass |

## Power Supply Products

| Vendor | Model Name | Product Description | Result |
| --- | --- | --- | --- |
| YHI | YS-6200DJ | 200W 110/ 220 ATX | Pass |
| SeaSonic | SS200GPX | 200W 110/ 220 ATX | Pass |

## DRAM Module

| Type | Vendor | Parts No | Description | Result |
| --- | --- | --- | --- | --- |
| Fast Page | Goldstar | GM71C18160AJ7 | 4MB 72pin RAM module 70ns | Pass |
|  |  |  | 8MB 72pin RAM module 70ns | Pass |
|  | Hyundai | HY514400A | 4MB 72pin RAM module 70ns | Pass |
|  |  | HY514400A | 8MB 72pin RAM module 70ns | Pass |
|  |  | HY514400A | 16MB 72pin RAM module 70ns | Pass |
|  | Hitachi | HM514400CLS7 | 4MB 72pin RAM module 70ns | Pass |
|  |  |  | 8MB 72pin RAM module 70ns | Pass |
|  | IBM | B101320BA-70 | 4MB 72pin RAM module 70ns | Pass |
|  |  | B014400J1E-70 | 4MB 72pin RAM module 70ns | Pass |
|  |  |  | 8MB 72pin RAM module 70ns | Pass |
|  |  | B014400J1D-70 | 8MB 72pin RAM module 70ns | Pass |
|  | Mitsubishi | M5M418160BJ-7 | 4MB 72pin RAM module 70ns | Pass |
|  |  | M5M4100BJ-7 | 4MB 72pin RAM module 70ns | Pass |
|  |  | FC1121 REV 1.0 | 8MB 72pin RAM module 70ns | Pass |
|  |  | M5M417400AJ-7 | 16MB 72pin RAM module 70ns | Pass |
|  |  | M5M417400BJ-7 | 16MB 72pin RAM module 70ns | Pass |
|  | Micron | MT-12D136G-7 | 4MB 72pin RAM module 70ns | Pass |
|  | NEC | UPD4217400LA-70 | 4MB 72pin RAM module 70ns | Pass |
|  |  | UPD424400LA-70 | 4MB 72pin RAM module 70ns | Pass |
|  |  | 4216100-60 | 64MB 72pin RAM module 60ns | Pass |
|  | Samsung | KM416C1200AJ-L7 | 4MB 72pin RAM module 70ns | Pass |
|  |  | KM44C1000BJ-7 | 4MB 72pin RAM module 70ns | Pass |
|  | TI | TMS418160-70DZ | 4MB 72pin RAM module 70ns | Pass |
|  |  | TMS417400DJ | 32MB 72pin RAM module 70ns | Pass |
|  | Toshiba | TC514400ASJL-7 | 4MB 72pin RAM module 70ns | Pass |
|  |  | TC5117400BSJ-60 | 32MB 72pin RAM module 70ns | Pass |
| EDO | SEC | KM416C1204AJ-60NS | 8MB 72pin RAM module 60ns | Pass |
|  |  | KM44C4104AK-60NS | 16MB 72pin RAM module 60ns | Pass |
|  | MICRON | MT4C4007JDJ-60 | 4MB 72pin RAM module 60ns | Pass |
|  |  | MT4C4007JDJ-70 | 4MB 72pin RAM module 70ns | Pass |
|  | TOSHIBA | M5117405A-60 | 16MB 72pin RAM module 60ns | Pass |
|  | SIEMENS | HYB5118165BSJ-60 | 4MB 72pin RAM module 60ns | Pass |
|  | NEC | 4217405-60 | 16MB 72pin RAM module 60ns | Pass |
|  | MITSUMI | M5M418165BJ-60 | 8MB 72pin RAM module 60ns | Pass |

# Software Compatibility Test Suite

## Base Operating System

The board is tested to ensure that it runs a complete a range of Operating Systems and thus operate in all network environments The board’s Power Management and Remote functions are also tested.

| Operating System | Version | Language | Company | Status |
| --- | --- | --- | --- | --- |
| MS DOS | 6.22 | English | Microsoft | Pass |
|  | 6.22 | German | Microsoft | Pass |
| Windows for Workgroups | 3.11 | English | Microsoft | Pass |
| IBM DOS | 7.0 | German | IBM | Pass |
| MS Windows 95 | 4.0a | English | Microsoft | Pass |
|  | 4.0a | Chinese | Microsoft | Pass |
| Windows NT Advance Server | 3.51 | English | Microsoft | Pass |
| Windows NT Workstation | 3.51 | English | Microsoft | Pass |
| IBM OS/2 Warp | 3.0 | English | IBM | Pass |
|  | 3.0 | German | IBM | Pass |
| ACAD 386 R13 |  | English | Autodesk | Pass |
| Novell Network | 3.12 | English | Novell | Pass |
| Novell Network | 4.10 | English | Novell | Pass |

## Software Applications

| Item |  | Descriptions |  | Version |  | Language |  | O.S. |  | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Suites | Lotus SMART Suite |  | 3.0 |  | English |  | Windows |  | Pass |  |
|  | Framework IV |  |  |  | English |  | DOS |  | Pass |  |
|  | Geoworks |  |  |  | English |  | DOS |  | Pass |  |
|  | MS Works for Windows |  | 3.0 |  | English |  | Windows |  | Pass |  |
|  | MS Works for Windows95 |  | 4.0 |  | English |  | Windows 95 |  | Pass |  |
|  | MS Office for Windows |  | 4.3 |  | English |  | Windows |  | Pass |  |
|  | MS Office for Windows |  | 4.3 |  | Chinese |  | Windows |  | Pass |  |
|  | MS Office for Windows95 |  | 7.0 |  | English |  | Windows 95 |  | Pass |  |
|  | Symphony |  | 3.0 |  | English |  | DOS |  | Pass |  |
| Word Processing | PE III (DW3) |  | 4.0 |  | English |  | DOS |  | Pass |  |
|  | AMI Pro for Windows |  | 3.1 |  | English |  | Windows |  | Pass |  |
|  | AMI Pro for Windows |  | 3.1 |  | Chinese |  | Windows |  | Pass |  |
|  | Word Pro for Windows96 |  |  |  | Chinese |  | Windows |  | Pass |  |
|  | MS Word for Windows |  | 6.0 |  | English |  | Windows |  | Pass |  |
|  | MS Word for Windows |  | 6.0 |  | Chinese |  | Windows |  | Pass |  |
|  | Word Prefect for Windows |  | 6.0 |  | English |  | Windows |  | Pass |  |
|  | WordPerfect |  | 6.0 |  | English |  | Windows |  | Pass |  |
| Presentations | MS PowerPoint |  | 4.0 |  | English |  | Windows |  | Pass |  |

|  | Descriptions | Version | Language | O.S. | Status |
| --- | --- | --- | --- | --- | --- |
| Spreadsheets | Borland Quatro | 3.0 | English | Windows | Pass |
|  | Louts 1-2-3 | 5.0 | English | Windows | Pass |
|  | Louts 1-2-3 for Windows |  | English | Windows | Pass |
|  | Improv | 2.1 | English | Windows | Pass |
|  | MS Excel | 5.0 | English | Windows | Pass |
|  | MS Excel | 5.0 | Chinese | Windows | Pass |
|  | Quattor Pro | 5.0 | English | Windows | Pass |
| Communication | cc: Mail/Add-on |  | English | Windows | Pass |
|  | Telix |  | English | Windows | Pass |
|  |  |  | Chinese | Windows | Pass |
|  | WinFAX Pro | 4.0 | English | Windows | Pass |
|  | WinFAX Pro for Win 95 | 4.0 | English | Windows | Pass |
|  | LapLink | 6.0 | English | Windows | Pass |
|  | Norton Anywhere | 1.0 | English | Windows | Pass |
| CAD | 3D Design Plus | 2.0 | English | Windows | Pass |
|  | AutoCAD | R13 | English | DOS | Pass |
|  | AUTOCAD | R13 | English | Windows | Pass |
|  | Animator Pro | 1.3 | English | DOS | Pass |
|  | ORCAD |  | English | DOS | Pass |
|  | 3D Studio | 4.0 | English | DOS | Pass |
|  | Micro Station | 5.0 | English | DOS/Windows | Pass |
|  | PCAD | 4.5 | English | DOS | Pass |
|  | OrCAD | 4.01 | English |  | Pass |
| Databases | Approach | 2.1 | Chinese | Windows | Pass |
|  | Notes | 3.0 | English | Windows | Pass |
|  | Notes | 3.0 | Chinese | Windows | Pass |
|  | DBASE III PLUS | 1.1 | English | DOS | Pass |
|  | DBASE IV | 2.0 | English | DOS | Pass |
|  | FoxBase Microsoft | 2.1 | English | Windows | Pass |
|  | FoxPro Microsoft | 2.6 | English | Windows | Pass |
|  | MS Access for Windows | 2.0 | English | Windows | Pass |
|  | MS Access for Windows | 2.0 | Chinese | Windows | Pass |
|  | Paradox for Windows |  | English | Windows | Pass |
|  | Clipper | 5.2 | English | DOS | Pass |
|  | Dbase | 5.0 | English | Windows | Pass |
|  | FUNCKY II |  | English | DOS | Pass |
|  | CodeBase |  | English | DOS | Pass |
|  | Paradox | 4.5 | English | Windows | Pass |
| Programs | MS Visual Basic | 3.0 | English | Windows | Pass |
|  | MS Visual C++ | 2.0 | English | DOS | Pass |
|  | Borland Turbo C++ | 4.0 | English | Windows | Pass |
|  | Turbo C++ | 3.1 | English | Windows | Pass |

| Item | Descriptions | Version | Language | O.S. | Status |
| --- | --- | --- | --- | --- | --- |
| Graphics | Corel Draw | 4.0 | English | Windows | Pass |
|  | Corel Draw | 6.0 | English | Windows | Pass |
|  | Windows Draw | 3.0 | English | Windows | Pass |
|  | Visio | 3.0 | English | Windows | Pass |
|  | MS PowerPoint | 4.0 | English | Windows | Pass |
|  | Designer | 4.1 | English | Windows | Pass |
|  | Freelance | 2.1 | English | Windows | Pass |
|  | Ventura for Windows |  | English | Windows | Pass |
|  | Visio HOME |  | English | Windows | Pass |
|  | Photo Shop Pro | 3.0 | English | Windows | Pass |
|  | ABC Flowcharter | 3.0 | English | Windows | Pass |
| Diagnostic | Bapco95 for Win NT | 1.0 | English | Windows | Pass |
| Programs | PC Magazine Benchmark 96 |  | English | Windows | Pass |
|  | Speed for Win95 | 3.0 | English | Windows | Pass |
|  | QAPlus | 5.20 | English | DOS | Pass |
|  | Landmark(Speedcom) |  | English | DOS | Pass |
|  | QAPlus | 4.8 | English | DOS | Pass |
| Tools | After Dark | 3.0 | English | DOS | Pass |
|  | AfterDark | 3.0 | English | Windows | Pass |
|  | AfterDark for Win95 | 3.0 | English | Windows | Pass |
|  | Central Point Backup | 1.0 | English | Windows | Pass |
|  | Desq View | 2.6 | English | Windows | Pass |
|  | Desq View386 | 2.6 | English | Windows | Pass |
|  | MS Testing Tools |  | English | Windows | Pass |
|  | Norton Backup | 2.2 | English | Windows | Pass |
|  | Norton Commander | 4.0 | English | DOS | Pass |
|  | Norton Desktop | 3.0 | English | Windows | Pass |
|  | Norton Utility | 8.0 | English | DOS | Pass |
|  | Norton Utility | 8.0 | English | Windows | Pass |
|  | PC Tools | 9.0 | English | DOS | Pass |
|  | QAPlus | 4.7 | English | DOS | Pass |
|  | Page Maker | 5.0 | English | Windows | Pass |
|  | MS-Plus 21Century | 4.0 | English | DOS | Pass |
|  | QEMM386 | 7.03 | English | DOS | Pass |
|  | Sidekick | 2.0 | English | DOS | Pass |
| Time | Project for Win95 | 4.1 | English | Windows | Pass |
| Management | Organizer | 2.1T | Chinese | Windows | Pass |
| Scanners | PC-Cillin | 4.0 | English | DOS | Pass |
|  | VB66 | 6.6 | English | DOS | Pass |

| Item | Descriptions | Version | Language | O.S. | Status |
| --- | --- | --- | --- | --- | --- |
| CD Games | Win Commander III Game |  | English | DOS | Pass |
| CD Titles- Multimedia | Cinderella |  | English | Windows | Pass |
|  | Living Book Grandma & Me |  | English | Windows | Pass |
|  | Top Gun |  | English | Windows | Pass |
|  | Multimedia Super Pack |  | English | Windows | Pass |
|  | Compton’s Interactive Encyclopedia |  | English | Windows | Pass |
|  | Adams Family |  | English | Windows | Pass |
|  | Blank Rain |  | English | Windows | Pass |
| CD Titles | Action | 3.0 | English | Windows | Pass |
|  | Calendar Address Book |  | English | Windows | Pass |
|  | The Ultrasound CD |  | English | Windows | Pass |
|  | Trip Maker |  | English | Windows | Pass |