Getting Started

Low Profile User Guide

- Installing Your Card.

- Installing Display Driver

- Troubleshooting Tips.

# Installing YourCard

Now that you have prepared your computer, you are ready to install your graphics accelerator card.

To install your graphics accelerator card.

- Power off the computer and displays, then disconnect the display cable from the back of your computer.

- Remove the computer cover. If necessary, consult your computer’s manual for help in removing the cover. Remember to discharge your body’s static electricity by touching the metal surface of the computer chassis.

- Remove any existing graphics card from your computer. If the old graphics card sticks, rock it gently from end to end. Remember to save the screw. Or if your computer has any on-board graphics capability, you may need to disable it on the motherboard. For more information, see your computer’s manual.

- If necessary, remove the metal cover from the empty AGP expansion slot; then, align your card with the AGP expansion slot, and press firmly until the card is fully seated. Grasp the card by the top edge and carefully seal it firmly into the appropriate expansion slot. Do not force the card – it should fit smoothly into the expansion slot.

- Replace the screw to fasten the card in place, and replace the computer cover.

- Plug the display cable into your card, then turn on the computer and display device. For cards with a Digital Flat Panel (DFP) connector, connect your monitor or flat panel display (or both) to the appropriate connector, as shown. Make sure your display cable is securely fastened.

You are now ready to install drivers. Select an enhanced driver from the following list:

- Put your Driver CD into CD-ROM Drive, and you will see the driver will be installed automatically.

- If there is no response on the monitor, please follow instructions as below:

- Click “ START ”

- Select “ RUN “

- Type the following: D:\SETUP.EXE or D:\AUTORUN.EXE

- If D is not your CD-ROM Drive, substitute D with the correct Drive letter.

# Troubleshooting Tips

The following troubleshooting tips may help if you experience problems. Contact your dealer for more advanced troubleshooting information.

- Check that the card is seated properly in its expansion slot.

- Ensure the display cable is securely fastened to the card’s display   connector.

- Make sure that the display and computer are plugged in and receiving power.

- If necessary, disable any built-in graphics capabilities on your motherboard. For more information, see your computer’s manual.

- Make sure you selected the appropriate display device and graphics card when you installed the enhanced driver.

# System Support

- Windows ME

- Windows 2000

- Windows XP