### Appendix A

Specifications

A.1	Hardware Specifications

Generic Classification

Video Display Adapter/Fixed Function Graphics Accelerator

Bus Type

32-bit AGP Bus

Video Connector

VGA Connector for CRT Monitor

S-video Connector for TV/VCR

DVI-I Connector for digital CRT or Flat panel

Graphics Accelerator

ATI RADEON™ 9800/9800 Pro

Display Memory

128MB DDR SDRAM

Monitor Compatibility

All VGA, Super VGA and Multisync.  The resolutions up to 2048x1536 and 1920x1440 require a multisync monitor with a 96 and 90K Hz maximum horizontal scan rate, respectively.

LCD Monitor Compatibility

All LCD monitor with DVI interface connector, the resolution upto 1600x1200.

A.2	Programming Information

The card supports all VGA modes.  This is the default configuration of the VGA BIOS.

The extended VGA graphics mode is invoked by the normal INT 10 mode call.  All the VGA extended modes can be invoked with the VESA defined select extended mode BIOS call.

A.3	Resolution Table

Resolutions and color depth table.

| GRAPHICS RESOLUTION | COLOR DEPTH |  |  | FRAME BUFFER SIZE |  |
| --- | --- | --- | --- | --- | --- |
| 640x480 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |
| 800x600 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |
| 1024x768 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |
| 1152x864 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |
| 1280x1024 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |
| 1600x1200 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |
| 1920x1080 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |
| 1920x1200 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |
| 1920x1440 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |
| 2048x1536 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |

8bpp  = 256 colors

16bpp = 64K colors = High color

32bpp = 16.7M colors = True color

The following table shows the resolutions for TV-Out:

| GRAPHICS RESOLUTION | COLOR DEPTH |  |  | FRAME BUFFER SIZE |  |
| --- | --- | --- | --- | --- | --- |
| 640x480 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |
| 800x600 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |
| 1024x768 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |

LCD resolutions and color depth table for R350.

| GRAPHICS RESOLUTION | COLOR DEPTH |  |  | FRAME BUFFER SIZE |  |
| --- | --- | --- | --- | --- | --- |
| 640x480 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |
| 800x600 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |
| 1024x768 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |
| 1152x864 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |
| 1280x1024 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |
| 1600x1200 | 8bpp | 16bpp | 32bpp | 32MB | 64MB |

My monitor is not capable of high resolution or refresh rate.

It depends on the display characteristics of your monitor.  Consult your monitor documentation for the proper configuration

This Page Left Blank for Note