### Chapter 3

Software Utility

The card is 100% VGA compatible and as such needs no special display drivers to run application software correctly at standard VGA resolutions and color depths.  However, enhanced display drivers are supplied with the VGA card because they provide accelerated performance, higher color depths and higher resolutions for software applications.  The VGA card supports resolutions as high as 2048x1536 pixels, and color depths of up to 32 bits per pixel.  Higher graphics resolutions and higher color depths provide greater clarity and detail when using the supplied drivers for graphics programs.

Driver Installation Procedure

3.1	Installing on Windows ME

General Installation Notes:

- Please install the AGP Driver provided by the motherboard company before you install VGA Driver.

- Before installing our user-friendly VGA configuration utility, please set up the VGA display as”standard PCI Graphics Adapter / Standard PCI Graphics Adapter( VGA )” mode.

- Microsoft DirectX allows 3D hardware acceleration support in Windows ME. For software MPEG support in Windows ME, you must first install the Microsoft DirectX, and then an MPEG Video Player.

- Start Windows.

- When Windows appears Add new Hardware Wizard dialog box,

Click Next until finish and then restart Windows.

- Insert the CD installation disc into your CD-ROM drive.

- The CD should auto run and the following screen will appear.

- When the Welcome box appears.  Click on the "Next" button to go to the next step.

- The License Agreement dialog box appears.  Click on “Yes” button.

- The select components dialog box appears, Click on “Express” Button.

- When the Windows SETUP has completed copying the files to the computer.  Windows will prompt you to restart Windows with the “Yes” option.  Then click on “Finish” button.

3.2	Installing on Windows 2000/XP

- Start Windows.

- When Windows detects your graphics card, the Found New Hardware Wizard dialog box appears.

- Click Cancel to enter the Windows desktop.

- Insert the CD installation disc into your CD-ROM drive.

- The CD should auto run and the following screen will appear.

- When the Welcome box appears.  Click on “Next" button to go to the next step.

- The License Agreement dialog box appears.  Click on “Yes” button.

- The select components dialog box appears, Click on “Express” Button.

- When the Windows SETUP has completed copying the files to the computer.  Windows will prompt you to restart Windows with the “Yes” option.  Then Click “Finish” button.

3.3		To enable/disable a display device

To make sure if your installation is successful, you can Right-click the Windows desktop and click Properties.  Click the Settings tab and then click Advanced.  The Radeon 9800 / 9800 pro Properties dialog box appears, Click Displays.

- Click the enable/disable  button for the display device you want to enable/disable.  A desktop image appears on the corresponding display device icon when the button is in the "Enable" position.

- Click either “OK” or “Apply” to save the changes you have made.

Note:

- At least one of your display devices must be enabled.

- At least one of your display devices must be set to the primary displays mode.

3.3.1	 	To adjust your monitor screen size

- Click the “Monitor” button.

Your monitor must be enabled before you can change its display properties.

- Click the “Adjustments” tab.

- Click the (+) and (-) buttons in the Vertical/Horizontal Screen area to change your screen size.

- The (+) and (-) buttons under Vertical Screen increase/decrease the vertical size of your monitor display.

- The (+) and (-) buttons under Horizontal Screen increase/decrease the horizontal size of your monitor display.

- Click “OK” or “Apply” to save the changes you have made.

3.3.2		To adjust your monitor synchronization

- Click here to view Display Properties at the Settings tab.

- Click the “Advanced” button.

- Click the “Displays” tab.

If you are running an extended desktop, you must first select the video adapter whose display properties you want to change.

- Click the “Monitor” button.

Your monitor must be enabled before you can change its display properties.

- Click the “Advanced” tab.

- In the Synchronization box, click the Horizontal or Vertical buttons to adjust synchronization.

Click either “OK” or “Apply” to save the changes you have made.

3.4	Dual Display Device Options

The following are sample display combinations for the card:

| - One RGB monitor and one DVI | - One RGB monitor and one TV (S-Video) |
| --- | --- |
| - One RGB monitor and one TV (Composite) |  |

3.5	Using the CRT/TV Features

3.5.1		How to use TV-out cables correctly

Caution!

Before you turn on your system, you have to be sure the VGA card is setup already and all the cables are connected.

DO NOT connect / disconnect TV-out cable when your TV or PC is Power On.

- Power off your PC and TV first.

- Connect the signal cable to the Video input at your TV from your VGA card, and then power on your PC and TV.

Turn Off

Turn Off

3.5.2		How to change TV setting

Your graphics accelerator is configured for TV display. Using this feature, you can connect a TV to your computer’s video output to view the desktop.

If you are not using a TV for your computer’s display, it is recommended that you disable TV display for optimal performance (higher refresh rates).  You can re-enable TV display at a later date.

- For NTSC, 1024x768x60Hz

- For PAL, 1024x768 x50Hz

Click here to view Display Properties at the Settings tab.

- Click the Advanced button.

- Click the Displays tab.

If you are running an extended desktop, you must first select the video adapter whose display properties you want to change.

- Click the button for the TV device.

Your TV must be enabled before you can change its display properties.

Note:

TV Mode (only)

NTSC: 1024x768x60Hz

PAL: 1024x768x50Hz

- Click the Advanced tab.

- In the S-Video Sharpness box, select a value from the list.

- Click OK or Apply to save the changes you have made.

To select your TV broadcast format by country/region

- Click here to view Display Properties at the Settings tab.

- Click the Advanced button.

- Click the Displays tab.

If you are running an extended desktop, you must first select the video adapter whose display properties you want to change.

- Click the TV button.

Your TV must be enabled before you can change its display properties.

- Click the Format tab.

- In the Country/Region box, select a country from the list.

- Click OK or Apply to save the changes you have made.

Note:

In general, there is only one broadcast format supported by a given country.

3.6  To add/remove color correction control points

- Click here to view Display Properties at the “Settings” tab.

- Click the “Advanced” button.

- Click the “Color” tab.

- In the Color Curve area, click the radio button corresponding to the desired color component (Red, Green or Blue)

- To add a control point, hold the “SHIFT” key down while clicking the mouse cursor at the desired point on the color curve. A control point appears on the color curve at the selected spot.

- To remove a control point, hold the “CTRL” key down while clicking the mouse cursor on the control point to be removed.  The control point disappears from the color curve.

- Click “OK” or “Apply” to save the changes you have made.

3.7   Verifying Win ME Driver Installation

- Click on "My Computer", then "Control Panel", and then, finally, "System".

- You should be in the "System Properties" window.  Click on the "Device Manager" tab.  From here, go down to "Display Adapters" and double click. The drivers associated with the display adapter should read: "Radeon 9800 / 9800 Pro".  If this is not the case, the drivers will need to be re-installed.

Uninstall Display Driver

- Click “Start”, and then point to Settings.

- Click Control Panel.

- Double-click the Add/Remove Programs icon.

- Click the Install/Uninstall tab.

- Click ATI Display Driver / ATI Control Panel from the list.

- Click Add/Remove button.

- The system will prompt you to restart your computer.  Click yes to restart.

3.8  Verifying  Win2000 / XP Driver Installation

- Right-Click on “ My Computer”, and Click “ Properties”.

- Click Hardware tab, then click “Device Manager” window.   From here, go down to "Display Adapters" and double click. The drivers associated with the display adapter should read: "Radeon 9800 / 9800 Pro".  If this is not the case, the drivers will need to be re-installed.

W

Uninstall Display Driver

- Click “Start”, and then point to Settings.

- Click Control Panel.

- Double-click the Add/Remove Programs icon.

- Click ATI Display Driver / ATI Control Panel from the list.

- Click Change/Remove button.

The system will prompt you to restart your computer.  Click yes to restart.

Windows 2000

Windows XP

This Page Left Blank for Note