Chapter 5

Entertainment PC 98

This chapter provides a summary of the key requirements for Entertainment PC 98.

If there is a conflict with requirements or recommendations made elsewhere in this guide, the items in this chapter have precedence for Entertainment PC 98. Unless a specific requirement or exception is defined in this chapter, the requirements defined in the “Basic PC 98” chapter apply for Entertainment PC 98.

Important: The system requirements defined in this guide provide guidelines for designing PC systems that will result in the optimal user experience with typical Windows-based applications running under either the Microsoft Windows or Windows NT Workstation operating systems. These design requirements are not the basic system requirements for running the Windows operating system.

Contents

Entertainment PC 98 System Requirements

Entertainment PC 98 Audio Requirements

Entertainment PC 98 Graphics Components

Entertainment PC 98 Video and Broadcast Components

Entertainment PC 98 References

Checklist for Entertainment PC 98

# Entertainment PC 98 System Requirements

This section summarizes the requirements for the basic components of Entertainment PC 98 systems. System requirements for computational capabilities and memory are the same for Entertainment PC 98 as for all PC 98 systems.

#### 1 Entertainment PC 98 system includes two USB ports, with at least one easily accessible connector

| ##### Required |  |  |
| --- | --- | --- |

The Entertainment PC 98 system must include two USB ports, with at least one USB connector in an easily accessible location (not on the rear panel). This is to support connection of auxiliary input devices such as game pads, joysticks, and track balls.

The recommended location for an easily accessible connector is to place a port on the front of the PC. The second connector for static connections to continuously used devices can be placed on the rear of the PC.

The USB implementation must meet the requirements defined in the “USB” chapter in Part 3 of this guide.

#### 2 Entertainment PC 98 system includes two IEEE 1394 ports, with at least one easily accessible connector

| ##### Required |  |  |
| --- | --- | --- |

The Entertainment PC 98 system must include two IEEE 1394 ports, with at least one IEEE 1394 connector in an easily accessible location (not on the rear panel) to support camcorders and other digital consumer-electronics devices.

The recommended location for an easily accessible connector is to place a port on the front of the PC. The second connector for static connections to continuously used devices can be placed on the rear of the PC.

The IEEE 1394 implementation must meet the requirements defined in the “IEEE 1394” chapter in Part 3 of this guide.

#### 3 Entertainment PC 98 system does not include ISA expansion slots

| ##### Required |  |  |
| --- | --- | --- |

For Entertainment PC 98, the system must not contain ISA expansion slots that can be accessed by the end user. The benefits of designing ISA-free systems include easier and more stable system configuration, lower support cost, and improved performance.

Any on-board legacy implementations, such as BIOS ROM, Super I/O, 8042 controllers, math coprocessors, and so on, must meet the requirements defined in the “Legacy Support” appendix in the References part of this guide.

#### 4 All Entertainment PC 98 input devices meet USB HID specifications

| ##### Required |  |  |
| --- | --- | --- |

Recommended: Wireless connection for all input devices.

For Entertainment PC 98, all keyboards, pointing devices, and game pads and their connections must be compliant with the USB Device Class Definition for Human Interface Devices, Version 1.0 or higher, and with the USB HID Usages Table, whether implemented as wired or wireless devices.

The game device must support at least four devices simultaneously, and its driver must support Microsoft DirectInput.

Note: For family-room PC design, a wireless connection is strongly recommended for the keyboard, pointing device, and game pad. For a pointing device, this requirement can be satisfied by using a remote-control pointing device or by using a wireless keyboard that includes a connector to enable a standard mouse to be attached. The ability to attach a standard two-button mouse is strongly encouraged, although the device itself does not need to be included with the PC.

For more information about input devices, see the “I/O Ports and Devices” chapter in Part 4 of this guide.

#### 5 Entertainment PC 98 includes remote-control pointing device

| ##### Recommended |  |  |
| --- | --- | --- |

If a remote-control pointing device is implemented on an Entertainment PC 98 system, its command structure must be compliant with the USB HID Usages Table.

There is no requirement for implementing an IR or RF remote-control pointing device.

If included with an Entertainment PC 98 system, the remote-control device must have the following controls:

| Label | Icon | Function |
| --- | --- | --- |
| Power | — | Toggle switch between On and Standby power state |
| Start | Windows flag (under Microsoft licensing agreement) | Display Start menu (same action as the keyboard Windows logo key) |
| — | — | Mouse pointer control, including left and right mouse buttons |

In addition, the following button is recommended for remote-control devices.

| Label | Icon | Function |
| --- | --- | --- |
| Cancel | — | Same as the keyboard ESCAPE key |
| Enter | — | Same as the keyboard ENTER key |
| Menu | — | Display application menus or toolbar (same as the keyboard F10 key) |

All buttons and any additional controls implemented by the independent hardware vendor (IHV) must comply with the current USB HID specifications, including HID usage-code specifications as defined in the USB HID Usages Table.

Note: When the industry adopts standards for wireless controls, these will be incorporated as Entertainment PC system requirements.

#### 6 All Entertainment PC 98 pointing devices support DirectInput and work simultaneously

| ##### Required |  |  |
| --- | --- | --- |

All pointing devices implemented in the Entertainment PC 98 system must have drivers that support Microsoft DirectInput. All pointing devices must also be able to correctly provide simultaneous input. This means that the pointing devices must not use competing protocols and that no pointing device is automatically disabled when another pointing device is in use. This also applies to game input devices in order to enable multiplayer gaming.

For information about implementing drivers that support simultaneous use of devices, see the Microsoft DirectX DDK.

#### 7 Entertainment PC 98 includes data/fax/voice modem that supports V.pcm

| ##### Required |  |  |
| --- | --- | --- |

Recommended: NDIS 5.0-supported dial-up high-speed communications.

The Entertainment PC 98 minimum requirement for modem support is an internal PCM data/fax/voice modem, with WDM support where relevant. If the modem does not support V.pcm, it must be capable of being upgraded by end users to support V.pcm. For more information, see the “Modems” chapter in Part 4 of this guide.

#### 8 Entertainment PC 98 includes DVD-ROM drive and DVD-Video playback

| ##### Required |  |  |
| --- | --- | --- |

The DVD-ROM device included in an Entertainment PC 98 system must meet the requirements defined for DVD in the “Storage and Related Peripherals” chapter in Part 4 of this guide. An Entertainment PC 98 system must also provide playback support for DVD-Video content, as defined in the “Video and Broadcast Components” chapter in Part 4 of this guide.

# Entertainment PC 98 Audio Requirements

High-quality audio is a key differentiating feature for Entertainment PC 98. Audio fidelity and functions must be significantly better than for traditional PCs and on par with consumer-electronics stereos.

Using IEEE 1394 for positional 3-D audio and connections to home-theater systems will enable more-realistic game and video experiences. One opportunity for achieving this is to implement the audio subsystem as an external digital-to-analog converter (DAC) attached to a secondary IEEE 1394 port on the rear of the PC. This isolates the analog audio stream from the RF noise of internal PC components while enabling easy connection to either legacy analog or new Plug and Play-compatible digital stereo components.

#### 9 Audio support meets PC 98 audio and Entertainment PC 98 requirements

| ##### Required |  |  |
| --- | --- | --- |

Recommended: Audio hardware accelerator is ready for digital audio.

Entertainment PC 98 audio must meet PC 98 audio requirements, which include requirements for audio hardware capabilities, performance metrics, and external connections. The following list summarizes the additional audio features required for Entertainment PC 98:

- Audio solution includes support for 3-D audio effects

- Input and output includes support for independent sample rates

- Audio solution supports music synthesis and echo cancellation

For more information, see the “Audio Components” chapter in Part 4 of this guide.

# Entertainment PC 98 Graphics Components

This section summarizes the Entertainment PC 98 requirements for graphics adapters and monitors. For complete information about the requirements summarized in this section, see the “Graphics Adapters” chapter in Part 4 of this guide.

#### 10 Graphics adapter uses AGP

| ##### Required |  |  |
| --- | --- | --- |

The graphics subsystem on an Entertainment PC 98 system must not use PCI, ISA, or VESA local bus (VLB). For information about implementing AGP, see the “Graphics Adapters” chapter in Part 4 of this guide.

Note: Integrated graphics subsystems that do not use AGP but meet or exceed AGP performance levels are acceptable for Entertainment PC 98.

#### 11 Entertainment PC 98 graphics subsystem includes PC 98 hardware acceleration for 2-D and 3-D graphics

| ##### Required |  |  |
| --- | --- | --- |

Windows and Windows NT operating systems provide application programming interfaces (APIs) that accelerate graphics display through direct manipulation of video display memory, hardware blters, hardware overlays, and page flipping. Hardware-acceleration features for 2-D and 3-D graphics must be implemented to improve overall graphics performance, as defined in the “Graphics Adapters” chapter in Part 4 of this guide.

#### 12 Entertainment PC 98 graphics subsystem includes support for television output if the system doesn’t have a large-screen monitor

| ##### Recommended |  |  |
| --- | --- | --- |

Support for NTSC, PAL, or both types of television output is recommended, except for systems bundled with a large-screen super VGA (SVGA) monitor.

For Entertainment PC 98, the ability to connect to a television is key to its ability to deliver more realistic television, movie, and game experiences, and to enable social computing activities. Television output integrated with the PC graphics adapter will deliver much higher image quality than external converters. As such, this feature optimizes the usability of an Entertainment PC system connected directly to a television in the family room and for desktop systems configured to transmit graphics and video to a television in another room.

This capability must meet the PC 98 requirements for television output for composite and S-Video connectors, parameter control, and hardware filtering and scaling capabilities as defined in the “Graphics Adapters” chapter in Part 4 of this guide.

#### 13 Entertainment PC 98 includes large-screen DDC2B color entertainment monitor

| ##### Recommended |  |  |
| --- | --- | --- |

A large-screen SVGA monitor that meets the Display Data Channel Standard, Version 2.0, Level B specification (DDC2B) is recommended for Entertainment PC 98 systems designed for the family room. Games, movies, and other entertainment software experiences are greatly enhanced by display screens comparable to modern television sizes—for example, 27 inches and larger in the United States.

An Entertainment PC 98 system that includes a large-screen monitor must meet the requirements for entertainment monitors defined in the “Monitors” chapter in Part 4 of this guide.

In addition, in 1999, large-screen entertainment monitors will likely be required to comply with interconnectivity standards currently in development as part of the VESA PC Theater initiative.

# Entertainment PC 98 Video and Broadcast Components

This section summarizes the Entertainment PC 98 hardware requirements for video capture, television output, and DVD playback support. For complete information about the requirements summarized in this section, see the “Video and Broadcast Components” chapter in Part 4 of this guide.

#### 14 Entertainment PC 98 DVD-Video and MPEG-2 playback meet PC 98 requirements

| ##### Required |  |  |
| --- | --- | --- |

DVD-Video support is required for Entertainment PC 98, so DVD playback capabilities must meet PC 98 requirements. If MPEG-2 hardware is included in an Entertainment PC 98 system, it must also meet PC 98 requirements.

#### 15 Entertainment PC 98 supports PC 98 analog video input and capture capabilities

| ##### Required |  |  |
| --- | --- | --- |

Recommended: Additional support for digital video input using IEEE 1394.

WDM support must be implemented for all video input and capture capabilities as part of the PC 98 requirements. The video input connector should be easily accessible on the Entertainment PC 98 system (that is, not on the rear panel)

#### 16 Entertainment PC 98 includes analog television tuner

| ##### Required |  |  |
| --- | --- | --- |

Recommended: Digital television tuner.

The NTSC or PAL decode component of the television tuner and analog video input subsystems must properly support extraction of data transmitted during the vertical blanking interval (VBI). This includes allowing certain scan lines to be placed within a separate memory buffer.

Note: The requirement for a cable-television tuner does not apply for PCs sold in Japan or other locales where cable television is not a common standard.

#### 17 Entertainment PC 98 includes digital broadcast satellite subsystem

| ##### Recommended |  |  |
| --- | --- | --- |

If this capability is included in Entertainment PC 98, the implementation must include a digital broadcast satellite network card, a smart card, and drivers that meet PC 98 requirements.

#### 18 Entertainment PC 98 includes DTV support

| ##### Required |  |  |
| --- | --- | --- |

Support for digital television (DTV) is required for Entertainment PC 98 starting in 1999, the expected time frame for initial digital television broadcasts. This includes hardware and software support for an ATSC tuner/demodulator, MPEG-2 decode capabilities, and graphics adapter support, as defined in the “Video and Broadcast Components” chapter in Part 4 of this guide.

The capability is required only for systems intended for use in the United States. As with all PC 98 components, compliance testing will begin when all related components are generally available.

# Entertainment PC 98 References

The following represents some of the references, services, and tools available to help build hardware that is optimized to work with Windows operating systems.

Display Data Channel Standard, Version 2.0 Level B (DDC2B)

Video Electronics Standards Association (VESA)
http://www.vesa.org

Windows and Windows NT DDKs

MSDN Professional membership

USB Device Class Definition for Human Interface Devices, Version 1.0
USB HID Usages Table
USB Implementers Forum

http://www.usb.org

# Checklist for Entertainment PC 98

If a recommended feature is implemented, it must meet the PC 98 requirements for that feature as defined in this document.

1.  Entertainment PC 98 system includes two USB ports, with at least one easily accessible connector

| Required |  |  |
| --- | --- | --- |

2.  Entertainment PC 98 system includes two IEEE 1394 ports, with at least one easily accessible connector

| Required |  |  |
| --- | --- | --- |

3.  Entertainment PC 98 system does not include ISA expansion slots

| Required |  |  |
| --- | --- | --- |

4.  All Entertainment PC 98 input devices meet USB HID specifications

| Required |  |  |
| --- | --- | --- |

5.  Entertainment PC 98 includes remote-control pointing device

| Recommended |  |  |
| --- | --- | --- |

6.  All Entertainment PC 98 pointing devices support DirectInput and work simultaneously

| Required |  |  |
| --- | --- | --- |

7.  Entertainment PC 98 includes data/fax/voice modem that supports V.pcm

| Required |  |  |
| --- | --- | --- |

8.  Entertainment PC 98 includes DVD-ROM drive and DVD-Video playback

| Required |  |  |
| --- | --- | --- |

9.  Audio support meets PC 98 audio and Entertainment PC 98 requirements

| Required |  |  |
| --- | --- | --- |

10.  Graphics adapter uses AGP

| Required |  |  |
| --- | --- | --- |

11.  Entertainment PC 98 graphics subsystem includes PC 98 hardware acceleration for 2-D and 3-D graphics

| Required |  |  |
| --- | --- | --- |

12.  Entertainment PC 98 graphics subsystem includes support for television output if the system doesn’t have a large-screen monitor

| Recommended |  |  |
| --- | --- | --- |

13.  Entertainment PC 98 includes large-screen DDC2B color entertainment monitor

| Recommended |  |  |
| --- | --- | --- |

14.  Entertainment PC 98 DVD-Video and MPEG-2 playback meet PC 98 requirements

| Required |  |  |
| --- | --- | --- |

15.  Entertainment PC 98 supports PC 98 analog video input and capture capabilities

| Required |  |  |
| --- | --- | --- |

16.  Entertainment PC 98 includes analog television tuner

| Required |  |  |
| --- | --- | --- |

17.  Entertainment PC 98 includes digital broadcast satellite subsystem

| Recommended |  |  |
| --- | --- | --- |

18.  Entertainment PC 98 includes DTV support

| Required |  |  |
| --- | --- | --- |

BLANK PAGE

IMPORTANT: This text will appear on screen, but will not print on a PostScript printer.

This page should be the last one in this file.

Do not type any additional text on this page!