New Key Support for Microsoftâ Windowsâ Operating Systems and Applications

(Optional Specification)

Specification :	9.2

Filename:	NEWKEYS.DOC

Date:		Februray 1, 1995

# Notice of Change

This version 1.2 of the New Keys Specification recommends a simpler, alternative make and break scan code response for the new Windowsâ and Application Logo Keys.  This recommendation is the result of feedback from keyboard manufacturers and our own internal testing.

The requirements for compatibility testing are not changed and a keyboard vendor can implement these new keys successfully following the 1.0, 1.1 or 1.2 versions of the New Keys Specification.  There is no change required for developers of Windows based applications to support any compatible implementation.

The Microsoft Natural Keyboardä which began shipping in September of 1994 follows the original 1.0 specification to implement the new Windows and Application Logo Keys.  The new codes described in this version of the New Keys Specification are meant to make it simpler to develop and test the implementation of the new keys.

Information in this document is subject to change without notice.  MICROSOFT MAKES NO WARRANTIES, EXPRESS OR IMPLIED, IN THIS DOCUMENT.

Microsoft, MS-DOS, Windows, Windows NT and Natural Keyboard are either registered trademarks or trademarks of Microsoft Corporation in the United States and/or other countries.

IBM is a registered trademark of International Business Machines.

ã 1995 Microsoft Corporation.  All rights reserved.

# Table of Contents

Notice of Change

Table of Contents

New Keys Support For Windows Software Applications

Key Codes For Scan Code Set 1:

Key Codes For Scan Code Set 2:

Key Codes For Scan Code Set 3:

What Software Applications Vendors Should Support And How

Suggestions For Support Of Application Key Combinations

Application Key Support In Microsoft Operating Systems

Using The Windows logo key Logo And Application Key Logo In Documentation

Windows logo key Support (Reserved For Operating System Use)

New Keys Support For OEMs And Keyboard IHVs

Software Support Of The New Keys In Windows 95

Microsoft Support Of The New Keys In Windows 3.1 / Windows NT And Windows 95

Special Keys Reserved For OEM Usage

Keyboard Detection In IntelliType 1.0

Licensing Intellitype Software From Microsoft

How To License Intellitype Software

Building A Keyboard With The New Keys

Tools Needed To Build A Keyboard With The New Keys

How To Get The Tools

.Steps To Ship A Keyboard With The New Windows Logo Keys

Retest Requirements

Recommended Physical Locations Of The New Keys

Recommendations For Laptop Keyboards

Appendix A:  Keyboards With New Windows logo key Scan Codes

General Requirements

For Scan Code Set 1:

For Scan Code Set 2:

For Scan Code Set 3:

Typematic Characteristics

Scan Code Assignments:

Note 1 For Scan Code 1:

Note 2 For Scan Code 2:

Note 3 Key 95 For Scan Codes 1 And 2:

Note 4 Key 124 For Scan Codes 1 And 2:

Note 5 Key 126 For Scan Codes 1 And 2:

Multiple Key Operation

Valid Combinations For LWIN, RWIN, And APP Keys:

Valid Final  Keys:

Valid 3-Key Combinations

Combinations of 3-Keys Which Are Unavailable:

Valid 2-Key Combinations

Additional General Requirements

Power On Reset Routine

Reset Command

New Keyboard Command Acknowledge

Hot Plug Requirements

LWIN and RWIN Keys as modifiers

# New Keys Support For Windows Software Applications

The three new keys report the following key codes in the Microsoftâ Windowsä  version 3.1, Windows NTä and Windowsâ 95 operating systems..

## Key Codes For Scan Code Set 1:

| New Key | Make | Break | Windows Virtual Key |
| --- | --- | --- | --- |
| Left Windows | E0 5B | E0 DB | 5B |
| Right Windows | E0 5C | E0 DC | 5C |
| Application | E0 5D | E0 DD | 5D |

## Key Codes For Scan Code Set 2:

| New Key | Make | Break | Windows Virtual Key |
| --- | --- | --- | --- |
| Left Windows | E0 1F | E0 F0 1F | 5B |
| Right Windows | E0 27 | E0 F0 27 | 5C |
| Application | E0 2F | E0 F0 2F | 5D |

## Key Codes For Scan Code Set 3:

| New Key | Make | Break | Windows Virtual Key |
| --- | --- | --- | --- |
| Left Windows | 8B | F0 8B | n/a |
| Right Windows | 8C | F0 8C | n/a |
| Application | 8D | F0 8D | n/a |

## What Software Applications Vendors Should Support And How

To provide support for the new keys, an application should support the application key virtual key scan code (5D) as a context menu event similar to a right mouse button click in some applications today.

Applications vendors are encouraged to extend their support beyond the application key to include support for application key combinations like Ctrl+Application, Alt+Application and Shift+Application.  These key combinations are reserved for applications to support.  Support for Windows+Application is reserved for the operating system.

| Application Key Combinations | Recommended Support |
| --- | --- |
| Ctrl + Application Key | unspecified, application specific |
| Alt + Application Key | unspecified, application specific |
| Shift + Application Key | unspecified, application specific |
| Windows + Application Key | reserved for operating system |
| Application Key + <alpha numeric keys> | not supported as a modifier key, support must be supplied by application |

### Suggestions For Support Of Application Key Combinations

- Context menu at the location of the text cursor (instead of the mouse pointer)

- Launch automated help agent

- Pop up list of last 5 actions (or list of commonly accessed functions)

- Enable/Disable macro recording or other user controllable features

- Switch to next open window in a multiple document

- User assignable key through an application UI

### Application Key Support In Microsoft Operating Systems

The application key will primarily function to bring up a context menu at the selection or mouse pointer.  This functionality is the same as the right mouse button click in some applications today.  To enhance the functionality of new keyboards with the Windows v. 3.1 or Windows NT v. 3.5 operating systems.  Microsoft has developed IntelliType software which is shipped with the Microsoft Natural Keyboard and can be licensed by OEMs or keyboard manufacturers for their keyboards.

|  | Application Key Response Supported By The Operating System |  |  |
| --- | --- | --- | --- |
| Operating System | w/o IntelliType Drivers | with IntelliType 1.0 | with IntelliType 1.1 |
| Windows 3.x | no response1 | no response1 | equals a Shift-F10 context menu3 |
| Windows NT 3.5x | no response1 | no response1 | equals a Shift-F10 context menu3 |
| Windows 95 | context menu on selection | n/a2 | context menu on selection |

1 Windows still reports a 5D virtual key code which means that applications can enable functionality

2 IntelliType 1.0 drivers are not compatible with Windows 95, IntelliType 1.1 will be available upon Windows 95 release

3 The IntelliType 1.1 software will simulate a right mouse button click if no other application hooks the 5D virtual key code

### Using The Windows logo key Logo And Application Key Logo In Documentation

The Windows Logo key and the Application Logo key may be used by OEMs, ISVs and IHVs in documentation that describes the functionality of the Logo keys, provided such description is consistent with Microsoft’s guidelines for use of the Logo keys.

•	It is not necessary to obtain a Logo license from Microsoft in order to refer to the Logo keys in documentation. This is the only permissible non-licensed use of the Windows Logo.

•	The Logos may not be used in any way other than as specified in the logo license guidelines.

•	Upon request, Microsoft will provide camera-ready artwork of the Logos to be used in documentation. Fax requests to Microsoft Windows Logo Department at 206-936-7329. Alternatively, you may use the Windows flag symbol provided in the “Wingdingsä” font. This character is available by pressing Alt + 0255 on the numeric keypad.

•	You may not alter the Logos in any way.

•	The Logos should be typed with an empty space before and after each symbol, followed by a plus sign and another space, followed by the modifier if appropriate. Examples are shown:

Windows logo key + B or   ÿ + B

Application key + S or   + + S

## Windows logo key Support (Reserved For Operating System Use)

The Windows logo keys are reserved for system level functions.  Software developers should not implement support for the Windows logo keys in Windows 95 based applications.  The Windows logo keys are supported in the operating system and provide system level functionality to the end user.  The following table lists a few of the Windows logo key combinations and their functionality.  This list is not exhaustive and additional combinations will be used by the Microsoft Windows 95 operating system.

| Windows logo key Combination | Functionality in Windows 95 |
| --- | --- |
| Windows + F1 | Display the popup menu for the selected object. |
| Windows + TAB | Activate next Taskbar buton. |
| Windows + E | Explore My Computer. |
| Windows + F | Find Document. |
| Windows + CTRL + F | Find Computer. |
| Windows + M | Minimize All. |
| SHIFT + Windows + M | Undo Minimize All. |
| Windows + R | Display Run dialog box. |
| Windows + PAUSE (Break) | Perform a system function. |

# New Keys Support For OEMs And Keyboard IHVs

OEMs that buy their keyboards with Windows logo keys from a licensed keyboard manufacturer and do not modify the keyboard product except to add their logo and other cosmetic changes are not required to license the Windows Logo Keys from Microsoft.  If the OEM product changes the layout or BIOS of an already licensed keyboard the OEM must go through the same licensing and certification procedure as described in the next section of this document.

## Software Support Of The New Keys In The Windows 95 Operating System

The new keys are supported in the Windows 95 user interface.  Application vendors will enable functionality for  the Application key in their applications.  10 Windows logo key combinations are reserved for OEM use.

| Key Event Supported | Windows 95 Support | Application Support | OEM System Utilities |
| --- | --- | --- | --- |
| Windows Logo Key | open the Start menu |  |  |
| Windows logo key Combinations | 9 shortcut functions |  | Win + 1-0 reserved for OEM |
| Application Logo Key | context menu on selection | context menu on selection | context menu on selection |
| Ctrl, Alt, Shift+Application |  | varies by application | varies by utility |

## Microsoft Support Of The New Keys In Windows 3.1 / Windows NT And Windows 95

OEMs can develop their own software which supports the new keys or they can license the IntelliType software that Microsoft has developed to support the new keys.

| Feature | IntelliType 1.0 | IntelliType 1.1 | Windows 95 w/o IntelliType |
| --- | --- | --- | --- |
| Windows logo keys bring up new task manager | ü | ü | Start Menu |
| Windows combination key support |  | 20 total | 9 total |
| Application key simulates right mouse button |  | ü | Context Menu |
| Right and left Windows logo key machine logon | ü (Windows NT only) | ü |  |
| Keyboard sound effects | ü | ü |  |
| Setable key lock states | ü | ü |  |
| Caps lock disable/enable | ü | ü |  |
| Mouse cursor control from the numeric keypad | ü | ü | ü |
| Fast move with Ctrl key and mouse keys | ü | ü |  |
| Pointer wrap mouse cursor | ü (Windows 3.1 only) | ü |  |
| Sonar locator for mouse cursor | ü | ü |  |
| Mouse cursor snap to default button | ü | ü |  |
| Mouse cursor vanish on typing |  | ü |  |

## Special Keys Reserved For OEM Usage

Ten Windows logo keys combinations have been reserved for OEM use.  These keys can be used by OEMs to provide keyboard hotkey controls for Speaker Volume, Monitor/LCD Brightness or Contrast, Password or other value added functions.

| Windows logo key Combination | Status |
| --- | --- |
| Windows + 1 | reserved for OEMs |
| Windows + 2 | reserved for OEMs |
| Windows + 3 | reserved for OEMs |
| Windows + 4 | reserved for OEMs |
| Windows + 5 | reserved for OEMs |
| Windows + 6 | reserved for OEMs |
| Windows + 7 | reserved for OEMs |
| Windows + 8 | reserved for OEMs |
| Windows + 9 | reserved for OEMs |
| Windows + 0 | reserved for OEMs |
| Windows + all other key combinations | reserved for operating system |

## Keyboard Detection In IntelliType 1.0

There is no keyboard detection requirement in the Windows 95 operating system since there is currently no Plug & Play specification for the 8042 PS/2 interface.  Only IntelliType 1.0 software incorporates any form of keyboard detection.

The IntelliType 1.0 software detects for the presence of a Microsoft Natural Keyboard before it enables the full feature set.  If the software is used on any other keyboard it will reduce it’s functionality to the standard keyboard support as in the Windows v. 3.1 operating system.

Microsoft has developed IntelliType 1.0a which is an OEM version of the software that does not include keyboard detection.  The Windows 95-compatible version of IntelliType, version 1.1 will not include a detection mechanism and can also be licensed by OEMs or keyboard IHVs that want to provide IntelliType functionality with their keyboards.

## Licensing Intellitype Software From Microsoft

Microsoft will license the IntelliType software to keyboard IHVs or OEMs that wish to provide support for the new keys using the software Microsoft has developed for the Microsoft Natural Keyboard.  There are two versions of the IntelliType software and the software has been localized to support 5 languages.

| Platform Supported | IntelliType 1.0a | IntelliType 1.1 (Available June, 1995) |
| --- | --- | --- |
| Windows 3.1 or Windows 3.11 | ü | ü |
| Windows for WorkGroups 3.1 or 3.11 | ü | ü |
| Windows NT 3.5 and 3.51 | ü | ü |
| Windows 95 |  | ü |
| Language Supported | IntelliType 1.0a | IntelliType 1.1 (Available June, 1995) |
| English | ü | June 1995 |
| German | ü | July 1995 |
| French | ü | July 1995 |
| Spanish | ü | July 1995 |
| Italian | ü | July 1995 |

IntelliType Documentation:  Microsoft IntelliType includes extensive On-line help and printed documentation that can also be distributed by the licensee.

### How To License Intellitype Software

For more information about licensing the IntelliType software.

Call the Microsoft OEM Sales Number at	1 800 325 1233  Option 3, then 2    U.S. only

# Building A Keyboard With The New Keys

## Tools Needed To Build A Keyboard With The New Keys

New Keys Specification Version 1.2, dated Feb 1995 is the latest.

Windows Logo Key License

Windows Logo Key Testing Software

### How To Get The Tools

The latest version of the New Keys Specification, the testing software and the logo license are always available electronically as NEWKEYS.ZIP.

| Electronic Service | How to find NEWKEYS.ZIP |
| --- | --- |
| On the Internet use ftp or the World-Wide-Web | (ftp://ftp.microsoft.com/PerOpSys/Win_News, |
|  | http://www.microsoft.com). |
| On The Microsoft Network | open Computers and Software\Software Companies\Microsoft\Windows 95\WinNews. |
| On CompuServe. | type GO WINNEWS |
| On Prodigy. | JUMP WINNEWS |
| On America Online. | use keyword WINNEWS |
| On GEnie | download files from the WinNews area under the Windows RTC. |

## Steps To Ship A Keyboard With The New Windows Logo Keys

- Develop your keyboard according to the tables in Appendix A of the New Keys Specification.

Compatibility Requirement:   Keyboards are required to be compatible at the Windows virtual key code level.

You can accomplish this by either:

- Developing a keyboard with the electronics and firmware that comply with the specification or by,

- Developing a software driver for use with Windows 3.1 and Windows 95 that maps the proper virtual key codes to keyboard events on your keyboard.

- Test your implementation using the Windows Logo Key Testing Software to ensure that single key, two key and three key combinations work with the Windows logo keys and report the proper virtual scan codes in Windows.

- Once you are satisfied that your keyboard passes the tests, run the full test suite and use it to generate a set of encoded test results that you will need to send along with 2 signed copies of the Windows Logo Key License to:

Microsoft Windows Logo Key Certification Test

Microsoft Corporation

One Microsoft Way

Redmond, WA 98052-6399

- You must also provide a list of the keyboard technologies/products that the particular test results apply to.  This should be reported in Appendix B of the Logo License Agreement.  Follow the format from the example table.

Example Reporting Table for Compatible Keyboard Hardware (Appendix B of Logo License Agreement)

| Model Name | Base Model w/o Windows logo keys | Product Model Number |
| --- | --- | --- |
| eg. KEYB101-W | KEYB101 | WN908-XX |
| eg. Super New Keyboard | new keyboard design | 555-55 |

When you want to add additional keyboards to the list of licensed product you must submit a new Appendix B and a disk with the new test results to the address above.

5.	Once the results have been verified, you will be sent the following

- a copy of the camera ready artwork for the Windows logo key Logo and the Application Key Logo that you can use to produce the legends for your keyboards.

- one copy of the signed Windows Logo Licenses for your records.

Notice:  If you plan on shipping product to the PRC, Korea or Taiwan you must also send notification to:

Windows Logo Department
Microsoft Corporation
One Microsoft Way
Redmond, WA 98052-6399

### Retest Requirements

Any time that you implement the new keys on a new keyboard technology you must go through this testing and certification procedure.  You will also need to submit a retest any time that you change the key layout or the BIOS firmware on a certified keyboard.

### Recommended Physical Locations Of The New Keys

For full travel desktop keyboards Microsoft recommends that the Windows logo keys be located near the current modifier keys like Ctrl, Shift and Alt since the Windows logo keys must also function as modifier keys.  The Application key can be located wherever it appropriately fits.  In the attached diagram the keys were placed on the bottom row on either side of the space bar.  By changing the current size of the space bar the new keys can be located in a similar position on a standard keyboard.

### Recommendations For Laptop Keyboards

Given the crowded nature of laptop keyboards it is difficult to add three new keys. A minimal implementation of the new keys would require the addition of one Windows logo key and the replacement of the Fn. key for the Application key.

The two Windows logo keys are not distinguished individually by the Windows 95 operating system and the IntelliType software only uses a Left Windows logo key + Right Windows logo key press for the Logon/Logoff function.

Design Requirements

- The Windows Logo Key needs to function as a modifier key (Ctrl, Shift and Alt)

- There can be no other icons or words associated with the Windows Flag trademark on the keycap

Design Options

- There does not need to be a Left and Right Windows logo key to get full functionality in Windows 95

- The Application key can be a dual function key and can be used to replace the Fn. key

# Appendix A:  Keyboards With New Key Scan Codes

## General Requirements

This keyboard must be downward compatible with an enhanced type 101 keyboard.  This keyboard must be fully compatible with the PS/2 Keyboard Controller command set.  It must support all 3 standard scan code sets.  Three additional scan codes are to be generated from the three new keys (Left Windows, Right Windows, and Application).

The virtual key codes described in the following tables are specifically for Windows operation only and are not generated by the keyboard directly.  The keyboard manufacturer will need to provide a Windows logo keyboard driver to support these three new virtual keys for use with the Windows v. 3.1 operating system.  The support will be built into a driver shipped with future versions of the Windows and Windows NT operating systems.

For the three  new scan code values, we will use the following:

## For Scan Code Set 1:

| New Key | Make | Break | Windows Virtual Key |
| --- | --- | --- | --- |
| Left Windows | E0 5B | E0 DB | 5B |
| Right Windows | E0 5C | E0 DC | 5C |
| Application | E0 5D | E0 DD | 5D |

## For Scan Code Set 2:

| New Key | Make | Break | Windows Virtual Key |
| --- | --- | --- | --- |
| Left Windows | E0 1F | E0 F0 1F | 5B |
| Right Windows | E0 27 | E0 F0 27 | 5C |
| Application | E0 2F | E0 F0 2F | 5D |

## For Scan Code Set 3:

| New Key | Make | Break | Windows Virtual Key |
| --- | --- | --- | --- |
| Left Windows | 8B | F0 8B | n/a |
| Right Windows | 8C | F0 8C | n/a |
| Application | 8D | F0 8D | n/a |

# Typematic Characteristics

Typematic is the term used for keys that will auto repeat after a specified time delay. In scan code 1 and scan code 2, key 126 is the only key that is not Typematic. (This key is the only one that does not send a break code as well). In scan code 3 each key can be inhibited from normal functioning ( make code, break code, and Typematic). The key can send a make code only, a break code only, or not be Typematic. Reference IBM PS/2 Hardware Technical Reference, Keyboards (101- and 102-Key) section.

Scan Code 3 Default States:

| key | state | key | state | key | state | key | state |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Typematic | 33 | Typematic |  |  | 97 | make only |
| 2 | Typematic | 34 | Typematic |  |  | 98 | make only |
| 3 | Typematic | 35 | Typematic |  |  | 99 | make only |
| 4 | Typematic | 36 | Typematic |  |  | 100 | make only |
| 5 | Typematic | 37 | Typematic |  |  | 101 | make only |
| 6 | Typematic | 38 | Typematic |  |  | 102 | make only |
| 7 | Typematic | 39 | Typematic |  |  | 103 | make only |
| 8 | Typematic | 40 | Typematic |  |  | 104 | make only |
| 9 | Typematic | 41 | Typematic |  |  | 105 | make only |
| 10 | Typematic | 42 | Typematic |  |  | 106 | make only |
| 11 | Typematic | 43 | Typematic | 75 | make only | 107 | make only |
| 12 | Typematic | 44 | make/break | 76 | Typematic | 108 | make only |
| 13 | Typematic | 45 | Typematic |  |  |  |  |
| 14 | Typematic | 46 | Typematic |  |  | 110 | make only |
| 15 | Typematic | 47 | Typematic | 79 | Typematic | 111 | make only |
| 16 | Typematic | 48 | Typematic | 80 | make only | 112 | make only |
| 17 | Typematic | 49 | Typematic | 81 | make only | 113 | make only |
| 18 | Typematic | 50 | Typematic |  |  | 114 | make only |
| 19 | Typematic | 51 | Typematic | 83 | Typematic | 115 | make only |
| 20 | Typematic | 52 | Typematic | 84 | Typematic | 116 | make only |
| 21 | Typematic | 53 | Typematic | 85 | make only | 117 | make only |
| 22 | Typematic | 54 | Typematic | 86 | make only | 118 | make only |
| 23 | Typematic | 55 | Typematic |  |  | 119 | make only |
| 24 | Typematic | 56 | Typematic |  |  | 120 | make only |
| 25 | Typematic | 57 | make/break | 89 | Typematic | 121 | make only |
| 26 | Typematic | 58 | make/break | 90 | make only | 122 | make only |
| 27 | Typematic |  |  | 91 | make only | 123 | make only |
| 28 | Typematic | 60 | make/break | 92 | make only | 124 | make only |
| 29 | Typematic | 61 | Typematic | 93 | make only | 125 | make only |
| 30 | make/break | 62 | make only | 94 | make only | 126 | make only |
| 31 | Typematic |  |  | 95 | make only |  |  |
| 32 | Typematic | 64 | make only | 96 | make only |  |  |

# Scan Code Assignments:

| key location | US key assignments | scan 1 make | scan 1 break | scan 2 make | scan 2 brake | scan 3 make | scan 3 break |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | ~ ` | 29 | A9 | 0E | F0 0E | 0E | F0 0E |
| 2 | ! 1 | 02 | 82 | 16 | F0 16 | 16 | F0 16 |
| 3 | @ 2 | 03 | 83 | 1E | F0 1E | 1E | F0 1E |
| 4 | # 3 | 04 | 84 | 26 | F0 26 | 26 | F0 26 |
| 5 | $ 4 | 05 | 85 | 25 | F0 25 | 25 | F0 25 |
| 6 | % 5 | 06 | 86 | 2E | F0 2E | 2E | F0 2E |
| 7 | ^ 6 | 07 | 87 | 36 | F0 36 | 36 | F0 36 |
| 8 | & 7 | 08 | 88 | 3D | F0 3D | 3D | F0 3D |
| 9 | * 8 | 09 | 89 | 3E | F0 3E | 3E | F0 3E |
| 10 | ( 9 | 0A | 8A | 46 | F0 46 | 46 | F0 46 |
| 11 | ) 0 | 0B | 8B | 45 | F0 45 | 45 | F0 45 |
| 12 | _ - | 0C | 8C | 4E | F0 4E | 4E | F0 4E |
| 13 | + = | 0D | 8D | 55 | F0 55 | 55 | F0 55 |
| 15 | Backspace | 0E | 8E | 66 | F0 66 | 66 | F0 66 |
| 16 | Tab | 0F | 8F | 0D | F0 0D | 0D | F0 0D |
| 17 | Q | 10 | 90 | 15 | F0 15 | 15 | F0 15 |
| 18 | W | 11 | 91 | 1D | F0 1D | 1D | F0 1D |
| 19 | E | 12 | 92 | 24 | F0 24 | 24 | F0 24 |
| 20 | R | 13 | 93 | 2D | F0 2D | 2D | F0 2D |
| 21 | T | 14 | 94 | 2C | F0 2C | 2C | F0 2C |
| 22 | Y | 15 | 95 | 35 | F0 35 | 35 | F0 35 |
| 23 | U | 16 | 96 | 3C | F0 3C | 3C | F0 3C |
| 24 | I | 17 | 97 | 43 | F0 43 | 43 | F0 43 |
| 25 | O | 18 | 98 | 44 | F0 44 | 44 | F0 44 |
| 26 | P | 19 | 99 | 4D | F0 4D | 4D | F0 4D |
| 27 | { [ | 1A | 9A | 54 | F0 54 | 54 | F0 54 |
| 28 | } ] | 1B | 9B | 5B | F0 5B | 5B | F0 5B |
| 29* | ¦ \ | 2B | AB | 5D | F0 5D | 5D | F0 5D |
| 30 | Caps Lock | 3A | BA | 58 | F0 58 | 14 | F0 14 |
| 31 | A | 1E | 9E | 1C | F0 1C | 1C | F0 1C |
| 32 | S | 1F | 9F | 1B | F0 1B | 1B | F0 1B |
| 33 | D | 20 | A0 | 23 | F0 23 | 23 | F0 23 |
| 34 | F | 21 | A1 | 2B | F0 2B | 2B | F0 2B |
| 35 | G | 22 | A2 | 34 | F0 34 | 34 | F0 34 |
| 36 | H | 23 | A3 | 33 | F0 33 | 33 | F0 33 |
| 37 | J | 24 | A4 | 3B | F0 3B | 3B | F0 3B |
| 38 | K | 25 | A5 | 42 | F0 42 | 42 | F0 42 |
| 39 | L | 26 | A6 | 4B | F0 4B | 4B | F0 4B |
| 40 | : ; | 27 | A7 | 4C | F0 4C | 4C | F0 4C |
| 41 | “ ‘ | 28 | A8 | 52 | F0 52 | 52 | F0 52 |
| 42** |  | 2B | AB | 5D | F0 5D | 53 | F0 53 |
| 43 | Enter | 1C | 9C | 5A | F0 5A | 5A | F0 5A |
| 44 | L SHIFT | 2A | AA | 12 | F0 12 | 12 | F0 12 |
| 45** |  | 56 | D6 | 61 | F0 61 | 13 | F0 13 |
| 46 | Z | 2C | AC | 1A | F0 1A | 1A | F0 1A |
| 47 | X | 2D | AD | 22 | F0 22 | 22 | F0 22 |
| 48 | C | 2E | AE | 21 | F0 21 | 21 | F0 21 |
| 49 | V | 2F | AF | 2A | F0 2A | 2A | F0 2A |
| 50 | B | 30 | B0 | 32 | F0 32 | 32 | F0 32 |
| 51 | N | 31 | B1 | 31 | F0 31 | 31 | F0 31 |
| 52 | M | 32 | B2 | 3A | F0 3A | 3A | F0 3A |
| 53 | < , | 33 | B3 | 41 | F0 41 | 41 | F0 41 |
| 54 | > . | 34 | B4 | 49 | F0 49 | 49 | F0 49 |
| 55 | ? / | 35 | B5 | 4A | F0 4A | 4A | F0 4A |
| 56*** |  | 73 | F3 | 51 | F0 51 | 51 | F0 51 |
| 57 | R SHIFT | 36 | B6 | 59 | F0 59 | 59 | F0 59 |
| 58 | L CTRL | 1D | 9D | 14 | F0 14 | 11 | F0 11 |
| 60 | L ALT | 38 | B8 | 11 | F0 11 | 19 | F0 19 |
| 61 | Space Bar | 39 | B9 | 29 | F0 29 | 29 | F0 29 |
| 62 | R ALT | E0 38 | E0 B8 | E0 11 | E0 F0 11 | 39 | F0 39 |
| 64 | R CTRL | E0 1D | E0 9D | E0 14 | E0 F0 14 | 58 | F0 58 |
| 75 | Insert | Note 1 | Note 1 | Note 2 | Note 2 | 67 | F0 67 |
| 76 | Delete | Note 1 | Note 1 | Note 2 | Note 2 | 64 | F0 64 |
| 79 | L Arrow | Note 1 | Note 1 | Note 2 | Note 2 | 61 | F0 61 |
| 80 | Home | Note 1 | Note 1 | Note 2 | Note 2 | 6E | F0 6E |
| 81 | End | Note 1 | Note 1 | Note 2 | Note 2 | 65 | F0 65 |
| 83 | Up Arrow | Note 1 | Note 1 | Note 2 | Note 2 | 63 | F0 63 |
| 84 | Dn Arrow | Note 1 | Note 1 | Note 2 | Note 2 | 60 | F0 60 |
| 85 | Page Up | Note 1 | Note 1 | Note 2 | Note 2 | 6F | F0 6F |
| 86 | Page Down | Note 1 | Note 1 | Note 2 | Note 2 | 6D | F0 6D |
| 89 | R Arrow | Note 1 | Note 1 | Note 2 | Note 2 | 6A | F0 6A |
| 90 | Num Lock | 45 | C5 | 77 | F0 77 | 76 | F0 76 |
| 91 | Numeric 7 | 47 | C7 | 6C | F0 6C | 6C | F0 6C |
| 92 | Numeric 4 | 4B | CB | 6B | F0 6B | 6B | F0 6B |
| 93 | Numeric 1 | 4F | CF | 69 | F0 69 | 69 | F0 69 |
| 95 | Numeric / | Note 3 | Note 3 | Note 3 | Note 3 | 77 | F0 77 |
| 96 | Numeric 8 | 48 | C8 | 75 | F0 75 | 75 | F0 75 |
| 97 | Numeric 5 | 4C | CC | 73 | F0 73 | 73 | F0 73 |
| 98 | Numeric 2 | 50 | D0 | 72 | F0 72 | 72 | F0 72 |
| 99 | Numeric 0 | 52 | D2 | 70 | F0 70 | 70 | F0 70 |
| 100 | Numeric * | 37 | B7 | 7C | F0 7C | 7E | F0 7E |
| 101 | Numeric 9 | 49 | C9 | 7D | F0 7D | 7D | F0 7D |
| 102 | Numeric 6 | 4D | CD | 74 | F0 74 | 74 | F0 74 |
| 103 | Numeric 3 | 51 | D1 | 7A | F0 7A | 7A | F0 7A |
| 104 | Numeric . | 53 | D3 | 71 | F0 71 | 71 | F0 71 |
| 105 | Numeric - | 4A | CA | 7B | F0 7B | 84 | F0 84 |
| 106 | Numeric + | 4E | CE | 79 | F0 79 | 7C | F0 7C |
| 107*** |  | 7E | FE | 6D | F0 6D | 7B | F0 7B |
| 108 | Numeric Enter | E0 1C | E0 9C | E0 5A | E0 F0 5A | 79 | F0 79 |
| 110 | Esc | 01 | 81 | 76 | F0 76 | 08 | F0 08 |
| 112 | F1 | 3B | BB | 05 | F0 05 | 07 | F0 07 |
| 113 | F2 | 3C | BC | 06 | F0 06 | 0F | F0 0F |
| 114 | F3 | 3D | BD | 04 | F0 05 | 17 | F0 17 |
| 115 | F4 | 3E | BE | 0C | F0 0C | 1F | F0 1F |
| 116 | F5 | 3F | BF | 03 | F0 03 | 27 | F0 27 |
| 117 | F6 | 40 | C0 | 0B | F0 0B | 2F | F0 27 |
| 118 | F7 | 41 | C1 | 83 | F0 83 | 37 | F0 37 |
| 119 | F8 | 42 | C2 | 0A | F0 0A | 3F | F0 3F |
| 120 | F9 | 43 | C3 | 01 | F0 01 | 47 | F0 47 |
| 121 | F10 | 44 | C4 | 09 | F0 09 | 4F | F0 4F |
| 122 | F11 | 57 | D7 | 78 | F0 78 | 56 | F0 56 |
| 123 | F12 | 58 | D8 | 07 | F0 07 | 5E | F0 5E |
| 124 | Print Screen | Note 4 | Note 4 | Note 4 | Note 4 | 57 | F0 57 |
| 125 | Scroll Lock | 46 | C6 | 7E | F0 7E | 5F | F0 5F |
| 126 | Pause | Note 5 | Note 5 | Note 5 | Note 5 | 62 | F0 62 |

Note *	Key 29 is available on the US and not on the International Keyboard.

Note **	Keys 42 and 45 are available on International and not on the US Keyboard.

Note ***	Keys 56 and 107 are not available on either the US or International Keyboards.

## Note 1 For Scan Code 1:

These keys may have additional “shift” and/or “unshift” scan codes preceding the Base Make code and following the Base Break code, depending upon the current state of Num Lock and the state of Shift key/s (when multiple keys are held down at the same time).

These Keys are Typematic

| Key Location | US key assignment | Base Make | Base Break |
| --- | --- | --- | --- |
| 75 | Insert | E0 52 | E0 D2 |
| 76 | Delete | E0 53 | E0 D3 |
| 79 | Left Arrow | E0 4B | E0 CB |
| 80 | Home | E0 47 | E0 C7 |
| 81 | End | E0 4F | E0 CF |
| 83 | Up Arrow | E0 48 | E0 C8 |
| 84 | Dn Arrow | E0 50 | E0 D0 |
| 85 | Page Up | E0 49 | E0 C9 |
| 86 | Page Down | E0 51 | E0 D1 |
| 89 | Right Arrow | E0 4D | E0 CD |

| Num Lock ON | Precede Base Make code with | follow Base Break code with |
| --- | --- | --- |
| Final Key only | E0 2A | E0 AA |
| LSHIFT down + Final Key |  |  |
| RSHIFT down + Final Key |  |  |
| Both LSHIFT and RSHIFT down + Final Key |  |  |

| Num Lock OFF | Precede Base Make code with | follow Base Break code with |
| --- | --- | --- |
| Final Key only |  |  |
| LSHIFT down + Final Key | E0 AA | E0 2A |
| RSHIFT down + Final Key | E0 B6 | E0 36 |
| Both LSHIFT and RSHIFT down + Final Key | E0 AA E0 B6 | E0 36 E0 2A |

As an example, consider the case where Num Lock is OFF and you choose to push and hold Left Shift, then push and hold Right Shift, and finally push Insert. The scan codes sent would be:

2A			(LSHIFT make)

36 			(RSHIFT make)

E0 AA E0 B6 E0 52	(Insert make)

When these keys are released, the keys still held down at the time a key is released determine it’s scan code.  Assume Insert is released first then Right Shift and last Left Shift. The scan codes would be:

E0 D2 E0 36 E0 2A	(Insert break)

B6			(RSHIFT break)

AA			(LSHIFT break)

## Note 2 For Scan Code 2:

These keys may have additional “shift” and/or “unshift” scan codes preceding the Base Make code and following the Base Break code, depending upon the current state of Num Lock and the state of Shift key/s (when multiple keys are held down at the same time).

These Keys are Typematic

| key location | US key assignment | Base Make | Base Break |
| --- | --- | --- | --- |
| 75 | Insert | E0 70 | E0 F0 70 |
| 76 | Delete | E0 71 | E0 F0 71 |
| 79 | Left Arrow | E0 6B | E0 F0 6B |
| 80 | Home | E0 6C | E0 F0 6C |
| 81 | End | E0 69 | E0 F0 69 |
| 83 | Up Arrow | E0 75 | E0 F0 75 |
| 84 | Dn Arrow | E0 72 | E0 F0 72 |
| 85 | Page Up | E0 7D | E0 F0 7D |
| 86 | Page Down | E0 7A | E0 F0 7A |
| 89 | Right Arrow | E0 74 | E0 F0 74 |

| Num Lock ON | Precede Base Make code with | follow Base Break code with |
| --- | --- | --- |
| Final Key only | E0 12 | E0 F0 12 |
| LSHIFT down + Final Key |  |  |
| RSHIFT down + Final Key |  |  |
| Both LSHIFT and RSHIFT down + Final Key |  |  |

| Num Lock OFF | Precede Base Make code with | follow Base Break code with |
| --- | --- | --- |
| Final Key only |  |  |
| LSHIFT down + Final Key | E0 F0 12 | E0 12 |
| RSHIFT down + Final Key | E0 F0 59 | E0 59 |
| Both LSHIFT and RSHIFT down + Final Key | E0 F0 12 E0 F0 59 | E0 59 E0 12 |

## Note 3 Key 95 For Scan Codes 1 And 2:

This Key is Typematic (Numeric / on US keyboards)

| Scan Code | base | L-SHIFT + base | R-SHIFT + base | L-SHIFT + R-SHIFT + base |
| --- | --- | --- | --- | --- |
| 1 make | E0 35 | E0 AA E0 35 | E0 B6 E0 35 | E0 AA E0 B6 E0 35 |
| 1 break | E0 B5 | E0 B5 E0 2A | E0 B5 E0 36 | E0 B5 E0 36 E0 2A |
| 2 make | E0 4A | E0 F0 12 E0 4A | E0 F0 59 E0 4A | E0 F0 12 E0 F0 59 E0 4A |
| 2 break | E0 F0 4A | E0 F0 4A E0 12 | E0 F0 4A E0 59 | E0 F0 4A E0 59 E0 12 |

## Note 4 Key 124 For Scan Codes 1 And 2:

This Key is Typematic (Print Screen on US keyboards)

| Scan Code | Make | Break | LCtrl or RCtrl + LSHIFT or RSHIFT + Make | LCtrl or RCtrl + LSHIFT or RSHIFT + Break | LAlt or RAlt + Make | LAlt or RAlt + Break |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | E0 2A E0 37 | E0 B7 E0 AA | E0 37 | E0 B7 | 54 | D4 |
| 2 | E0 12 E0 7C | E0 F0 7C E0 F0 12 | E0 7C | E0 F0 7C | 84 | F0 84 |

## Note 5 Key 126 For Scan Codes 1 And 2:

This Key is not Typematic and has no Break Code (Pause on US keyboards)

| Scan Code | Make | L-Ctrl or R-Ctrl + Make |
| --- | --- | --- |
| 1 | E1 1D 45 E1 9D C5 | E0 46 E0 C6 |
| 2 | E1 14 77 E1 F0 14 F0 77 | E0 7E E0 F0 7E |

# Multiple Key Operation

This keyboard must respond identically to an IBM enhanced 101 keyboard with reguard to the combination key presses that are supported.  However; since the three new scan codes are not supported on an enhanced 101 keyboard, the following new rules apply for multiple key response to the three new scan codes:

## Valid Combinations For LWIN, RWIN, And APP Keys:

| LWIN | RWIN | APP |
| --- | --- | --- |
|  |  | X |
|  | X |  |
|  | X | X |
| X |  |  |
| X |  | X |
| X | X |  |

The following tables describe the valid two and three key combinations for this keyboard.  Invalid Key combinations will generate an Key Error code.  For each two and three key combinations, the following table applies for the final key  in the sequence.

## Valid Final  Keys:

|  |  |  |  | Esc | Tab | c | Left Arrow |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | F1 | q | v | Right Arrow |  |
|  |  |  |  | F2 | w | b | Up Arrow |  |
|  |  |  |  | F3 | e | n | Down Arrow |  |
|  |  |  |  | F4 | r | m |  |  |
|  |  |  |  | F5 | t | , | APP |  |
|  |  |  |  | F6 | y | . | Enter |  |
|  |  |  |  | F7 | u | / | Print Screen |  |
|  |  |  |  | F8 | i | Insert | Scroll Lock |  |
|  |  |  |  | F9 | o | Delete | Pause |  |
|  |  |  |  | F10 | p | Home | Num Lock |  |
|  |  |  |  | F11 | [ | End | Numeric 0 |  |
|  |  |  |  | F12 | ] | Page Up | Numeric 1 |  |
|  |  |  |  | ` | \ | Page Down | Numeric 2 |  |
|  |  |  |  | 1 | a |  | Numeric 3 |  |
|  |  |  |  | 2 | s |  | Numeric 4 |  |
|  |  |  |  | 3 | d |  | Numeric 5 |  |
|  |  |  |  | 4 | f |  | Numeric 6 |  |
|  |  |  |  | 5 | g |  | Numeric 7 |  |
|  |  |  |  | 6 | h |  | Numeric 8 |  |
|  |  |  |  | 7 | j |  | Numeric 9 |  |
|  |  |  |  | 8 | k |  | Numeric / |  |
|  |  |  |  | 9 | l |  | Numeric * |  |
|  |  |  |  | 0 | ; |  | Numeric - |  |
|  |  |  |  | - | ‘ |  | Numeric + |  |
|  |  |  |  | = | z |  | Numeric Enter |  |
|  |  |  |  | Backspace | x |  | Numeric . |  |
|  |  |  | Indicates Traditional IBM Final keys. |  |  |  |  |  |

## Valid 3-Key Combinations

The following table defines the valid 3-key combinations.  An “X” indicates that key is selected.  Each Row is followed by a 3rd key from the previous “Valid Final Keys” Table.

| LWIN | RWIN | APP | LCTRL | RCTRL | LSHIFT | RSHIFT | LALT | RALT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X |  |  | X |  |  |  |  |  |
| X |  |  |  | X |  |  |  |  |
| X |  |  |  |  | X |  |  |  |
| X |  |  |  |  |  | X |  |  |
| X |  |  |  |  |  |  | X |  |
| X |  |  |  |  |  |  |  | X |
|  | X |  | X |  |  |  |  |  |
|  | X |  |  | X |  |  |  |  |
|  | X |  |  |  | X |  |  |  |
|  | X |  |  |  |  | X |  |  |
|  | X |  |  |  |  |  | X |  |
|  | X |  |  |  |  |  |  | X |
| X | X |  |  |  |  |  |  |  |
|  |  |  | X |  | X |  |  |  |
|  |  |  | X |  |  | X |  |  |
|  |  |  |  | X | X |  |  |  |
|  |  |  |  | X |  | X |  |  |
|  |  |  | X |  |  |  | X |  |
|  |  |  | X |  |  |  |  | X |
|  |  |  |  | X |  |  | X |  |
|  |  |  |  | X |  |  |  | X |
|  |  |  |  |  | X |  | X |  |
|  |  |  |  |  | X |  |  | X |
|  |  |  |  |  |  | X | X |  |
|  |  |  |  |  |  | X |  | X |

|  |  |  | Indicates Traditional IBM 3-Key combinations. |
| --- | --- | --- | --- |

## Combinations of 3-Keys Which Are Unavailable:

Microsoft has designed a key switch layout for the Microsoft Natural Keyboard. Based upon this layout, several keys will be “ghosted”, meaning not uniquely detectable by the firmware during the key switch scanning of 3-Key combinations.  The following table details the key combinations  which are “ghosted” keys and therefore are excluded from the above tables.

| 1st Key | 2nd Key | 3rd Key |
| --- | --- | --- |
| LWIN | RWIN | Up Arrow |
| RWIN | LALT | key 56 |
| RWIN | RALT | key 42 |
| RWIN | RCTRL | Caps Lock |
| RWIN | LCTRL | Caps Lock |
| LWIN | LALT | key 107 |
| LCTRL | LSHIFT | Pause, Caps Lock |
| RCTRL | RSHIFT | Pause, Caps Lock |

## Valid 2-Key Combinations

The following table defines the valid 2-key combinations.  An “X” indicates that key is selected.  Each Row is followed by a 2nd key from the previous “Valid Final Keys” Table.

| LWIN | RWIN | APP | LCTRL | RCTRL | LSHIFT | RSHIFT | LALT | RALT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X |  |  |  |  |  |  |  |  |
|  | X |  |  |  |  |  |  |  |
|  |  | X |  |  |  |  |  |  |
|  |  |  | X |  |  |  |  |  |
|  |  |  |  | X |  |  |  |  |
|  |  |  |  |  | X |  |  |  |
|  |  |  |  |  |  | X |  |  |
|  |  |  |  |  |  |  | X |  |
|  |  |  |  |  |  |  |  | X |

There are no exceptions to this 2-Key Combination table.

# Additional General Requirements

## Power On Reset Routine

During a Power On Reset condition, this keyboard must perform a self-test and respond with  a diagnostic completion code  (AA for pass, otherwise return an error code).  Completion codes must be sent within 450 msecs and 2.5 seconds after the Power On Reset.  After Reset, this keyboard defaults to scan code set 2.

## Reset Command

Upon receiving a FF (soft Reset command) from the host, this keyboard responds with a completion code,  within 300 and 500 msecs after acknowledging the Reset command.

## New Keyboard Command Acknowledge

All keyboard commands are acknowledged with an FA (ACK) or FE (Re-send).  The Acknowledge response time must be within 20 msecs.

## Hot Plug Requirements

This keyboard must  be designed to allow plugging and unplugging into a host computer while the host computer is under power.  This action shall not cause any destructive effects on the host computer.   This keyboard must then continue normal operation as if a Power On Reset Condition  occurred.

## LWIN and RWIN Keys as modifiers

This keyboard must generate unique scan codes for make and break.  The LWIN and RWIN keys must be treated as modifier keys, much in the same way as Alt, and Control keys are handled today.  It will be up to the operating system to decide on an implementation scheme for the modifier key functionality.