Daniela's ATAPI Disk Filter - Release 0.2.3

-------------------------------------------

"Christmas Edition"

NAME

DaniADsk.FLT  -  replacement for IBMATAPI.FLT

SYNOPSIS

This filter driver supports all of the ATAPI disks that IBMATAPI does

(LS-120, ZIP-100, JAZZ), and in addition the ZIP-250 (including full media

swapping).

DESCRIPTION

This release of my ATAPI DASD filter driver is based on the IBMATAPI.FLT

sources.

My enhancements are:

- can deal with the ATAPI ZIP-250

- supports arbitrary swapping of 100MB and 250MB media

- reflects the current media capacity up to FDISK, DISKIO or the like

***  - OS/2 boot from ZIP is possible

***  - faster 32bit PIO in conjunction with DANIS506.ADD (rel. 1.1.1 or later)

Current implementation status:

- it suffers from the same bugs as IBMATAPI.FLT

except for

* trap at APM resume when the base S506 driver exports

forced ATAPI fake devices

* hang during boot for slave devices

COMMAND LINE OPTIONS

This driver supports all switches and options IBM1ATAPI.FLT supports.

***  To enable booting OS/2 from an ATAPI ZIP drive, I introduced the new

/ZA switch ('map ZIP to A:'). It takes effect only if the BIOS has mapped

the ZIP to drive letter A: in order to boot from it; in all other cases

the /ZA option is a no-op.

If there are floppy disk drives in your system, then - as a side effect -

/ZA remaps floppy drive A: to B:, and floppy drive B: (if present) is

no longer accessable. Due to limitations in OS2DASD.DMD (?) floppy

reads and writes are a little 'rocky' in this case.

A boot from ZIP automatically locks the boot medium in the drive, media

swapping is no longer possible! This lock is imposed automatically by

OS2DASD. If you insist to eject the locked medium by the 'eject' command,

this command obviously is routed into nirvana and causes a kernel trap.

INSTALLATION

Just copy DaniADsk.FLT to bootdrive:\OS2\BOOT and rename IBMATAPI.FLT to

DaniADsk.FLT in your CONFIG.SYS.

If you prefer chasing icons, you can do the copy using DDINSTAL; you

still have to tidy up CONFIG.SYS!

FILES

DaniADsk.FLT  The driver itself.

DaniADsk.DDP  The device driver profile for DDINSTAL

DaniADsk.DOC  The file you are reading right now :-)

cmprssd\DaniADsk.FLT

The same as above, but LXlite compressed. This one is *not*

guaranteed to work on every system.

BUGS

Hopefully none, but who knows ...

TO DO

- merge with the IDE CD filter driver

- merge with the base S506 driver

COPYRIGHT

DaniADsk - Copyright (c) 1999 Daniela Engert. All rights reserved.

STATUS OF PROGRAM

DaniADsk is freeware.

DISCLAIMER

YOU ARE USING THIS PROGRAM AT YOUR OWN RISK! I don't take any

responsibillity for damages, problems, custodies, marital dis-

putes, etc. resulting from use, inability to use, misuse, pos-

session or non-possession of this program directly or indirectly.

I also don't give any warranty for bug-free operation, fitness for

a particular purpose or the appropriate behaviour of the program

concerning animals, programers and little children.

THE SOFTWARE IS PROVIDED ``AS IS'' AND WITHOUT ANY EXPRESS OR

IMPLIED WARRANTIES, INCLUDING, WITHOUT LIMITATION, THE IMPLIED

WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

PURPOSE. THE ENTIRE RISK AS TO THE QUALITY AND PERFORMANCE OF THE

PROGRAM IS WITH YOU.  SHOULD THE PROGRAM PROVE DEFECTIVE, YOU

ASSUME THE COST OF ALL NECESSARY SERVICING, REPAIR OR CORRECTION.

Or, in a few words:

If its good, I am responsible.

If its bad, its all your fault. ;)

Permission is granted to redistribute this program free of charge,

provided it is distributed in the full archive with unmodified

contents and no profit beyond the price of the media on which it

is distributed is made. Exception to the last rule: It may be

included on freeware/shareware collections on CD-ROM, as well as

on magazine cover CD-ROMs.

All trademarks mentioned anywhere around her are property of their

owners and the like ...

AUTHOR

Daniela Engert

Internet: "Daniela Engert" <dani@ngrt.de>  (preferred)

Fidonet: 2:2490/2576

EPILOGUE

Suggestions and bug-reports are always welcome. Well ... bug-reports

are perhaps not *that* welcome ... ;)