MON51

A

Software Debug Monitor

For the 8051

Copyright 1991,1992 Dave Dunfield

All rights reserved

MON51                                                            Page: 1

1. INTRODUCTION

MON51 is an interactive software  debugger  and  machine  language

monitor for the Intel 8051 family  of  microprocessors.  It  contains

display/alter memory/register facilities, as  well  as  a  full  8051

disassembler, breakpoint, and software single step functions.

MON51  requires  no  hardware  assist,  all  functions   including

single-step are implemented in the MON51 firmware. MON51 communicates

with  the  user  via  the  internal   serial   port   of   the   8051

microcontroller, although  the  RDCHR  and  WRCHR  subroutines  could

easily be modified to support communications through an external UART

device.

MON51 requires 80 bytes of external  read/write  memory,  for  its

buffers and save areas, and about 3700 bytes of ROM  memory  for  the

program. If MON51 is placed in an 4K (2732 or compatable)  rom,  this

will leave over 300 bytes of space for user I/O routines.  The  MON51

rom must be at the bottom of the  8051  memory  map,  as  it  has  to

control the 8051 interrupt vectors.

MON51 preserves the entire contents of registers and internal  RAM

when passing control back and forth to the user program. This  allows

the user program to  operate  as  if  it  were  running  "standalone"

without fear of conflicting with MON51. TIMER1 of the 8051 is used by

MON51 both as the baud rate generator for serial communications,  and

as a interrupt source when single stepping. All other  8051  hardware

is available to the user program.

MON51                                                            Page: 2

2. USING MON51

2.1 Monitor Commands

The following commands are implemented in the monitor:

A <address>

Alter INTERNAL memory. MON51 prompts  with  the  specified

address and its current contents. You may  enter  TWO  hex

digits to change its value, SPACE to advance to  the  next

location, BACKSPACE to backup to the previous location, or

CARRIAGE RETURN to terminate the Alter command.

B <breakpoint#> <address>

Set breakpoint at specified address. Breakpoint is removed

if address is 0000. There can be up to  four  breakpoints,

which are referenced by the numbers 0-3.

If  SPACE  is  entered  instead  of  a  breakpoint#,   the

currently set breakpoint addresses are displayed.

NOTE: The 8051 family of processors does not have a single

byte transfer instruction such  as  is  normally  used  to

implement  breakpoints.   Breakpoints   are   handled   by

inserting 'LCALL' instructions into your code  during  the

processing of a 'G' command and restoring the code in  the

breakpoint handler. Each 'LCALL' occupies three  bytes  of

memory, which causes the following restrictions when using

breakpoints:

You MUST be careful to place breakpoints  in  locations

where there will NOT be  any  JUMPs  or  CALLs  to  the

addresses containing the second and third bytes of  the

breakpoint. For example, if you  set  a  breakpoint  at

address 1234, there should  NOT  be  a  label  in  your

program occuring at address 1235 or 1236 (Note, a label

at 1234 is OK).

You may not set breakpoints that are within three bytes

of  each  other.  The  message  'Breakpoint   conflict'

results if you attempt to do so.

Attempt to 'G'o at an address containing  a  breakpoint

will also result in the 'Breakpoint conflict'  message.

This will most commonly occur when you wish  to  resume

execution following a breakpoint.  In  this  case,  you

must either remove the breakpoint, or  use  the  'S'tep

command to advance the program counter until it is  NOT

positioned over any part  of  a  breakpoint  (Remember,

breakpoints are three bytes long).

MON51                                                            Page: 3

C <register> <value>

Changes  8051  registers  values.  Register  is  a  single

character, which may be as follows:

A   - Set Accumulator       ( 8 bit value).

B   - Set B register        ( 8 bit value).

D   - Set DPTR              (16 bit value)

S   - Set stack pointer     ( 8 bit value).

P   - Set program counter   (16 bit value).

W   - Set PSW               ( 8 bit value)

0-7 - Set R0-R7 in current register bank (8 bit value)

D <start>,<end>

Displays  EXTERNAL  memory,  in  HEX/ASCII  dump   format,

starting at the indicated address. If a SPACE  is  entered

for <end> address, assumes FFFF.

E <address>

Edit's  EXTERNAL  memory,   Address   and   contents   are

displayed, Enter TWO hex digits to change value.  Entering

SPACE skips to the next location, BACKSPACE backups to the

previous location. CARRIAGE  RETURN  terminates  the  edit

command.

F <start>,<end> <value>

Fill's external memory from <start> to <end> with the byte

<value>.

G <address>

Begins execution at the indicated address. If a  SPACE  is

entered instead of an address,  begins  execution  at  the

address in the 8051 program counter.

I <start>,<end>

Displays the  contents  of  INTERNAL  memory  between  the

specified addresses.

L

Downloads data from the console  port,  which  may  be  in

either MOTOROLA or INTEL hex format.

If you accidently enter this command, you may enter either

'S9' or ':00' to signify a null download file  and  return

to the command prompt.

MON51                                                            Page: 4

R

Displays the current values of the 8051 registers  (A,  B,

DPTR, SP, PC, PSW and R0-R7).

S

Single-Stepps  one  instruction  from  the  current   8051

Program Counter address. Disassembly  of  the  instruction

stepped is displyed on the console.

U <start>,<end>

Un-assembles memory, starting  at  indicated  address.  If

SPACE  is  entered  for  <end>  address,   assumes   FFFF.

Disassembler  output  contains  address,  opcodes   bytes,

instruction neumonic, and operands to instruction.

?

Displays a short help summary of the commands.

MON51                                                            Page: 5

2.2 Display Output

In order to maintain the maximum amount of data on the terminal

screen during the debugging session, MON51 has  been  designed  to

minimize the number of screen lines in any display.  All  commands

which output only one line of data (such as 'B' and 'S'), will  do

so on the same line on which the command is issued (To  the  right

of the command).

For commands which output larger amounts of data (such as  'D',

'U'), pressing the SPACE key during  the  display  will  halt  the

display on the next line. Each subsequent SPACE entered will cause

one more line of data be displayed. Pressing CARRIAGE RETURN  will

allow the display to continue at full speed.

The escape key may be used to cancel command output, and return

to the '*' prompt.

2.3 Interrupts

MON51 re-vectors all interrupts  (except  for  TIMER1  OVERFLOW

which is used internally for single-stepping) to the corresponding

locations in the first page of user RAM.

When debugging any program which uses  the  interrupt  vectors,

simply origin it at the  beginning  of  user  RAM,  and  code  the

interrupt handlers in exactly the same locations (relative to  the

beginning of the  program)  as  you  would  if  the  program  were

origined at location zero.

MON51

TABLE OF CONTENTS

Page

1. INTRODUCTION                                                        1

2. USING MON51                                                         2

2.1 Monitor Commands                                                2

2.2 Display Output                                                  5

2.3 Interrupts                                                      5