Willem's EpromProgrammer Tester 0.5  11/15/97

==================================

What it is

----------

This Utility was developed to debug and fix hardware problems with my Epromprogrammer.

You can set all control and data signals separatly and also read the two input signals.

SET LP1 or LPT2

SET DipSwitch to 27C64

test1:check printercable connection

----------------------------------------

set all signals separatly and check with a Voltage meter on the Centronics Connector on the PCB.

Connect 5V to both input 10 and 11 and read ,also connect both inputs to ground and read.

test2:check signal levels on the PCB. (follow steps in this order!)

-----------------------------------------

1.Set Power to high ,the LED should be on and VCC(5V) must be on Programmer Socket  32 and 30.

PIC Socket 14,24CXX Socket 8.

2.set ProgramPower to high, 12V program voltage should be on Programmer Socket 1and PICsocket 4.

3.set ProgramPulse to high,VCC should be on  Programmer Socket  31 ,29 and 24Cxx 7

4.set Read/Write to high ,pin 1 on IC 1 and IC 2(40097) should become high. pin 15 on IC2 should be low.

Programmer Socket  24 should become low.

5.set ReadClock to high,pin 10 on 4014 should be low.

6.set adressClock to high,pin 1 and 9 on all 4015 should be high, (except IC6 pin9),PIC socket pin 13 should be high.

7.set adressData to high,pin 15 on IC4(4015) should become high,pin 9 on IC7(4014) should

become high

8.set Read/write to low, PICsocket 12 and 24Cxx 6 should become high.

The hardware test is a very simple one;in fact it only tests for the board to be

connected to the computer and the power supply connected.

The signal goes from Centronics 2(Adres Clock) through IC2(40097) to IC3(4049) to

Q5 ,to Q4 ,to IC3 again to Centronics 11(input).So you can also test this with the

Signal setting test program.

The most common mistake is to forget solderings onm the component side,so check

for this very carefully.Also check for shortcuts.

Did you mount all programming IC sockets?

R12 on the component drawing says r12=10k this should be 100K.